MAVLink 2.0 Attack Detection

Mathematical Derivation of Detection Feature Formulas

How ts_gap and gps_sys_delta are computed, and how each attack's
injected timestamp produces its specific feature signature

Author: Jatin Thakur
Project: MAVLink 2.0 Drone Security Lab
Source files: detection/features.py, attacks/attack1-4*.py

# 1. Purpose of This Document

This document derives, step by step, the exact mathematical formulas used by detection/features.py to compute the two detection features (ts_gap and gps_sys_delta), and shows how each attack script's own constants flow through those formulas to produce the extreme values that the detector and the planned ML classifier rely on. Every constant shown here is taken directly from the project source code — none are estimated or rounded for convenience.

The purpose is to make explicit that the synthetic training data planned for the ML classifier is not arbitrary: the mean of each attack class's distribution is the literal output of the same formula the attack script itself uses to compute its injected timestamp.

# 2. Base Detection Formulas

## 2.1  Signature Timestamp Gap — ts_gap

MAVLink 2.0 signs every packet with a 6-byte timestamp field, counted in units of 10 microseconds since the MAVLink epoch (January 1, 2015, 00:00:00 UTC). The MAVLink epoch constant is:

MAVLINK_EPOCH = 1,420,070,400

The raw 6-byte field (raw_ts_value) is converted back to Unix seconds as:

sig_timestamp_unix = MAVLINK_EPOCH + raw_ts_value100,000

The division by 100,000 converts 10-microsecond units into whole seconds (10 microseconds x 100,000 = 1,000,000 microseconds = 1 second). The detector then compares this claimed timestamp against the real wall clock:

ts_gap = sig_timestamp_unix − time.time()

A positive ts_gap means the packet claims to be signed in the future relative to the detector's clock. Normal traffic has ts_gap close to zero (small negative values from network transmission delay).

## 2.2  GPS Clock Delta — gps_sys_delta

GPS_INPUT JSON packets carry a time_usec field — the GPS-reported time in microseconds since the Unix epoch. It is converted to seconds as:

gps_unix = time_usec1,000,000

This is then compared to the system wall clock the same way ts_gap is:

gps_sys_delta = gps_unix − time.time()

Both formulas have the identical shape: (claimed time) minus (real time). Every attack below works by choosing a raw_ts_value or time_usec that forces this subtraction to a huge number.

# 3. Attack 1 / Attack 4 — Future Timestamp Injection

Source: attacks/attack1_signed_cmd.py. The script defines a constant number of days to push the signed command's timestamp into the future:

FUTURE_DAYS = 1

The injected timestamp is computed as:

future = now + (FUTURE_DAYS × 86,400)

86,400 is the number of seconds in one day. Substituting FUTURE_DAYS = 1, this is the value sent as sig_timestamp_unix. Plugging into the base ts_gap formula (Section 2.1), with sig_timestamp_unix = future and time.time() = now:

ts_gap = future − now = (now + 86,400) − now = 86,400

ts_gap resolves to exactly 86,400 seconds, independent of when the packet is actually sent — the "now" terms cancel algebraically. The only deviation from exactly 86,400 in real traffic is a few milliseconds of network and processing delay between when the attacker computed "now" and when the detector reads it.

Attack 4 (Replay) reuses the raw bytes captured from Attack 1, so it produces the identical ts_gap = 86,400 signature — it is detected by the same formula, plus the duplicate-hash check.

# 4. Attack 2 — GPS Clock and Position Spoof

Source: attacks/attack2_gps_spoof.py. The script defines the clock advance in days:

SPOOF_DAYS = 10

Converted to seconds:

SPOOF_OFFSET_SEC = SPOOF_DAYS × 86,400 = 10 × 86,400 = 864,000

The spoofed GPS time injected into the packet is:

spoofed_unix = now + SPOOF_OFFSET_SEC

Substituting into the base gps_sys_delta formula (Section 2.2), with gps_unix = spoofed_unix:

gps_sys_delta = spoofed_unix − now = (now + 864,000) − now = 864,000

Same cancellation as Attack 1 — the result depends only on SPOOF_DAYS, not on wall-clock time. The script additionally drifts the reported latitude south by 0.0018 degrees per packet over 30 seconds, which produces the drift_m_per_s feature (not part of the 2-feature ML vector, but used for diagnostic logging).

# 5. Attack 3 — Timestamp Overflow (48-bit Maximum)

Source: attacks/attack3_overflow_dos.py. The MAVLink signing timestamp field is 48 bits wide, so its largest representable value is:

MAX_48BIT = 248 − 1 = 281,474,976,710,655

The script works backwards from this maximum: instead of choosing a date and computing the timestamp, it chooses the maximum possible timestamp and converts it back to a Unix date using the inverse of the base ts_gap formula:

OVERFLOW_UNIX = MAVLINK_EPOCH + MAX_48BIT100,000

OVERFLOW_UNIX = 1,420,070,400 + 248 − 1100,000

OVERFLOW_UNIX = 4,234,820,167   (Unix time for March 13, 2104)

This becomes the time_usec sent in the GPS_INPUT packet, so:

gps_sys_delta = OVERFLOW_UNIX − now ≈ +2,810,000,000   (≈ 78 years ahead in 2026)

Because this is the literal ceiling of the 48-bit field, the firmware's signing clock is forced to its absolute maximum. Every subsequent packet with a real 2026 timestamp is smaller than this value and is permanently rejected — this is what makes Attack 3 an irreversible denial-of-service rather than a temporary one.

# 6. Attack 3B — Timestamp Underflow (Pre-Epoch Date)

Source: attacks/attack3b_underflow_dos.py. The script injects a GPS date before the MAVLink epoch:

UNDERFLOW_UNIX = 1,262,304,000   (Unix time for January 1, 2010)

Because UNDERFLOW_UNIX is smaller than MAVLINK_EPOCH (January 1, 2015), the computed signing timestamp is negative when expressed as a signed integer:

MAV_TS_SIGNED = UNDERFLOW_UNIX − MAVLINK_EPOCH × 100,000

MAV_TS_SIGNED = 1,262,304,000 − 1,420,070,400 × 100,000 = −15,776,640,000,000

The firmware stores this timestamp as an unsigned 64-bit integer (uint64_t). A negative value cast to uint64 wraps around by adding 2^64:

MAV_TS_WRAPPED = 264 + MAV_TS_SIGNED

MAV_TS_WRAPPED = 18,446,744,073,709,551,616 − 15,776,640,000,000 = 18,446,728,297,069,551,616

This wrapped value is larger even than Attack 3's overflow value, producing the same style of permanent denial-of-service if the firmware stores it directly. For the gps_sys_delta feature read by the detector, the relevant comparison is simply against the injected (pre-epoch) Unix date itself:

gps_sys_delta = UNDERFLOW_UNIX − now ≈ −160,000,000   (strongly negative)

The exact magnitude depends on the current date (it grows as "now" advances further from 2010), but it is always a large negative number — the opposite sign from every other attack, which is itself a distinguishing signature.

# 7. Summary of All Derived Values

| Attack | Feature | Formula Source | Resulting Value |
| --- | --- | --- | --- |
| ATK1 / ATK4 | ts_gap | FUTURE_DAYS x 86,400 | +86,400 s |
| ATK2 | gps_sys_delta | SPOOF_DAYS x 86,400 | +864,000 s |
| ATK3 | gps_sys_delta | MAVLINK_EPOCH + (2^48-1)/100,000 - now | +2,810,000,000 s |
| ATK3B | gps_sys_delta | UNDERFLOW_UNIX - now | -160,000,000 s (approx.) |
| NORMAL | ts_gap | Measured SITL clock drift | -4.33 s (mean), 2.95 s (std) |
| NORMAL | gps_sys_delta | Measured SITL clock drift | -0.001 s (mean), 0.007 s (std) |

# 8. Connection to the Planned Synthetic Training Dataset

Each class's distribution mean in the planned classifier training set is not an arbitrary or observed number — it is the direct algebraic output of the formula shown for that attack above. The synthetic generator should compute the mean for each class using the same expression the attack script itself evaluates, then add a small standard deviation to represent real-world transmission jitter:

| Class | Mean (from formula above) | Suggested Std (jitter) |
| --- | --- | --- |
| NORMAL | -4.33 (ts_gap) / -0.001 (gps_sys_delta) | 2.95 / 0.007 (measured) |
| ATK1_FUTURE_TS | 86,400 (FUTURE_DAYS x 86,400) | ~5 seconds |
| ATK2_GPS_SPOOF | 864,000 (SPOOF_DAYS x 86,400) | ~5 seconds |
| ATK3_OVERFLOW | 2,810,000,000 (derived from 2^48-1) | ~1,000 seconds |
| ATK3B_UNDERFLOW | -160,000,000 (UNDERFLOW_UNIX - now) | ~1,000 seconds |

Because every class mean is separated from its neighbors by at least five to six orders of magnitude, a small jitter standard deviation cannot cause class overlap — a classifier trained on this data is expected to separate all five classes with near-perfect accuracy.

Two detection cases remain outside this magnitude-based scheme and are not covered by this formula derivation: ATK3B_BURST (a packet-rate pattern over a time window, not a single packet's feature value) and ATK4_REPLAY's duplicate check (a SHA-256 hash lookup, not a learnable numeric pattern). These two should remain as deterministic rule checks alongside the ML classifier.

End of Document