Internship Daily Report

# May 27, 2026

Goal: Initialize ArduPilot SITL environment on Arch Linux with MAVProxy and PyMAVLink, and prepare for Gazebo integration.

## Work Completed

Environment Setup

- Initialized drone simulation environment on Arch Linux.

- Cloned ArduPilot repository and configured SITL (Software In The Loop).

- Successfully built ArduCopter firmware, patched for GCC 16 compatibility.

- Configured Python virtual environment with all required dependencies (OpenCV, MAVProxy, etc.).

MAVLink & Ground Control

- Installed MAVProxy and PyMAVLink for MAVLink communication.

- Verified SITL launch with Map and Console modules functional.

- Installed xterm for simulation window management.

## Issues Faced & Resolutions

- Gazebo installation failure: AUR package (Gazebo Classic) failed due to broken dependencies (ruby2.7) and checksum errors on Arch Linux.

- Compiler incompatibility: ArduPilot build failed because GCC 16 treats warnings as errors. Resolved by patching boards.py to disable specific -Werror flags.

- Missing GUI dependencies: MAVProxy maps and console were missing modules (cv2, PIL). Resolved by installing via pip.

- Sudo timeouts: Background installation processes failed because password prompts timed out during long builds.

## Next Steps

- Transition to Ubuntu 22.04 for native, stable support for Gazebo Classic 11 and industry-standard robotics toolchains.

# May 28, 2026

Goal: Run ArduPilot SITL + Gazebo Harmonic + MAVProxy + Wireshark, all launched automatically from a single Python script (sitl_basic.py).

Stack: ArduPilot SITL v4.8.0-dev · Gazebo Harmonic (gz-sim 8.x) · ardupilot_gazebo plugin · MAVProxy 1.8.74 · Wireshark 3.6.2 · Python 3.10.12

## Work Completed

SITL Debugging

- The -N flag means no-rebuild, not no-MAVProxy — it was causing two MAVProxy instances to conflict on TCP 5760.

- The arducopter binary was launching inside a hidden minimised xterm; fixed by running it directly in a visible gnome-terminal.

- Added a while-loop retry to MAVProxy so it survives SITL's two-phase boot cycle (parameter load → reboot).

Gazebo Integration

- Correct SITL flag is --model JSON (not --model X) to enable JSON-based FDM communication with Gazebo.

- Fixed a lock_step=1 deadlock where Gazebo and SITL were each waiting on the other; resolved by starting SITL first, then Gazebo after a 12s delay.

- Set GZ_SIM_SYSTEM_PLUGIN_PATH and GZ_SIM_RESOURCE_PATH pointing to ~/ardupilot_gazebo/build and models/worlds.

Frame: UNSUPPORTED Fix

- Stale eeprom.bin in the ArduCopter directory was overriding --defaults, causing the wrong FRAME_TYPE.

- Fixed by deleting eeprom.bin and mav.parm before each launch, then passing --defaults pointing to copter.parm + gazebo-iris.parm to set FRAME_CLASS=1, FRAME_TYPE=1 (QUAD/X).

Wireshark MAVLink Monitoring

- Generated a 3.5 MB MAVLink Lua dissector from the pymavlink ardupilotmega dialect.

- Patched 99 occurrences of base.HEX_DEC → base.DEC for Wireshark 3.6 compatibility.

- Fixed dumpcap Permission Denied using sg wireshark -c to force the wireshark group for the session.

- Wireshark now captures and decodes live MAVLink packets on UDP 14550.

Python Launcher Script

- sitl_basic.py opens all 4 tools in separate gnome-terminal windows in the correct order: SITL → Gazebo (after 12s) → MAVProxy (retry loop) → Wireshark.

- Supports --kill to stop all processes cleanly and --no-gz to skip Gazebo for fast testing.

## System Architecture

Gazebo Harmonic  <-- UDP 9002 (JSON FDM) -->  arducopter binary

|

TCP 5760

|

MAVProxy console

|

UDP 14550 output

|

Wireshark (packet inspector)

## First Successful Flight

Ran mode GUIDED → arm throttle → takeoff 10. The drone climbed to 10 m and hovered. The Gazebo 3D view updated in real time, and Wireshark confirmed live HEARTBEAT, ATTITUDE, and GLOBAL_POSITION_INT packets.

## Next Steps

- Add tshark (Window 5) for a text-based live MAVLink stream.

- Fly a full autonomous waypoint mission using wp load.

- Connect QGroundControl to UDP 14550 as an alternative GCS.

- Explore other Gazebo worlds (iris_warehouse.sdf, iris_runway.sdf).

- Add a second drone instance using the -I1 flag.

# May 29, 2026

Goal: Simulate two GPS spoofing attacks on the MAVLink 2.0 protocol inside the SITL environment, capture packet evidence in Wireshark, and organise all work into a structured project folder.

Stack: ArduPilot SITL v4.8.0-dev · Gazebo Harmonic (gz-sim 8.x) · ardupilot_gazebo plugin · MAVProxy 1.8.74 · Wireshark 3.6.2 · Python 3.10.12 · pymavlink 2.4.49

## Work Completed

Project Organisation

- Created drone_security_lab/ project folder with subfolders: simulation/, attacks/, results/, reports/.

- Moved sitl_basic.py into simulation/ and created individual attack scripts for each of the 4 paper attacks.

Attack 1 — Signed Message Clock Manipulation

- Replicated Ficco et al. Stage 1 from the paper (Section IV-C-2a).

- Crafted a MAVLink 2.0 LAND command signed with key = 'key' and a timestamp set +1 day in the future.

- Sent via UDP 14550 (MAVProxy output port) — TCP 5760 is owned by MAVProxy and cannot be shared.

- UAV received the signed command; Wireshark captured Frame 130 (COMMAND_LONG) and Frame 131 (COMMAND_ACK).

- 765 packets captured; attack packet confirmed with future timestamp 2026-05-30 (+1 day).

Attack 2 — GPS Timestamp Spoofing

- Simulated Algorithm 1 from the paper — injected fake GPS_INPUT messages with timestamp +10 days.

- Configured MAVProxy: param set GPS1_TYPE 14, SIM_GPS1_DISABLE 1, module load GPSInput.

- Sent 150 GPS_INPUT packets at 5 Hz over 30 seconds to UDP port 25100.

- Wireshark decoded time_usec = 2026-06-08 16:00:08 IST, confirming the 10-day clock shift.

- 1,596 total packets captured; 150 attack packets + 1,446 normal MAVLink telemetry messages.

Wireshark MAVLink Dissector Fix

- Added GPS_INPUT_time_usec_fmt as a new string ProtoField in mavlink.lua.

- The new field converts raw microseconds to a human-readable date (e.g. 2026-06-08 16:00:01 UTC).

- Used as a custom Wireshark column so the spoofed date is visible directly in the packet list.

Documentation

- Generated attack_report.docx — full technical report with methodology, Wireshark evidence, and findings.

- All pcap evidence saved: attacks/results/attack1_capture.pcap and attack2_capture.pcap.

## Issues Faced & Resolutions

- Signing while armed error: ArduPilot refuses signing setup when motors are armed. Resolution: always run 'signing setup key' before arming, not after.

- TCP 5760 connection refused: the attack script tried to connect directly to SITL, but MAVProxy already owns that port. Resolution: changed the script to connect via UDP 14550 (MAVProxy's broadcast output).

- No ACK received after Attack 1: SITL bypasses signature enforcement over local USB/loopback connections. Resolution: expected behaviour per the paper — the clock manipulation effect still applied, confirmed by the Wireshark COMMAND_ACK frame.

- Wireshark column showing raw microseconds: the custom column showed 1780914608042893 instead of a date. Resolution: added a new GPS_INPUT_time_usec_fmt ProtoField (string) in mavlink.lua that formats the value as a UTC date.

- gps_time_format.lua Lua error: the post-dissector referenced a field before it was registered. Resolution: deleted the separate Lua file and moved the formatting directly into mavlink.lua.

## System Architecture

Gazebo  <-- UDP 9002 (JSON FDM) -->  arducopter binary

|

TCP 5760 (MAVLink)

|

MAVProxy console

|

UDP 14550 (output)

+-------------+-------------+

|                           |

Wireshark                  Attack scripts

(packet inspector)     (inject via UDP 14550 / 25100)

## Next Steps

- Attack 3 — Timestamp Overflow DoS: spoof GPS time to Unix 4,234,820,167 (48-bit max) to permanently brick MAVLink communication.

- Attack 4 — Replay Attack: capture a signed packet during a spoofed session, restart the UAV without key rotation, then replay to confirm acceptance.

- Document all 4 attacks with combined Wireshark analysis and add a countermeasures section.