UAV Security Research — Attack Report

Timestamp Manipulation-Based GPS Spoofing on MAVLink 2.0

Date: 29 May 2026   |   Time: 16:33   |   Platform: ArduPilot SITL + Gazebo Harmonic

# 1.  Overview

This report documents the simulation of four timestamp manipulation-based GPS spoofing attacks on the MAVLink 2.0 protocol, as described in the IEEE Transactions on Communications paper by Colton, Oracevic, and Dilek (2026). The attacks were reproduced in a fully controlled Software-in-the-Loop (SITL) environment using ArduPilot, Gazebo Harmonic, MAVProxy, and Wireshark.

All attacks exploit MAVLink 2.0's timestamp management rules — specifically the trust placed in GPS-derived time — to manipulate the UAV's internal signing clock without requiring knowledge of the secret key.

# 2.  Lab Environment

| Component | Tool / Version | Role |
| --- | --- | --- |
| Flight Controller Sim | ArduPilot SITL v4.8.0-dev | Simulated UAV running ArduCopter firmware |
| 3D Physics World | Gazebo Harmonic gz-sim 8.x | Visual and physics simulation of iris drone |
| GZ Plugin | ardupilot_gazebo (built) | Bridges SITL and Gazebo via JSON on UDP 9002 |
| Ground Control | MAVProxy 1.8.74 | Command console — sends MAVLink to UAV |
| Packet Capture | Wireshark 3.6.2 + tshark | Captures and decodes live MAVLink traffic |
| Attack Scripts | Python 3.10 + pymavlink | Crafts and injects malicious MAVLink packets |
| OS | Ubuntu 22.04 LTS | Host operating system |

## 2.1  Network Port Layout

Gazebo  <-- UDP 9002 (JSON FDM) -->  arducopter binary

|

TCP 5760  (MAVLink)

|

MAVProxy console

|

UDP 14550  (GCS output)

|

Wireshark  (captures all MAVLink here)

Attack scripts (inject via UDP 14550 / 25100)

# 3.  Launching the Simulation Stack

A single Python launcher script (sitl_basic.py) was used to open all simulation windows automatically in the correct startup order.

## 3.1  Startup Order and Timing

| Time | Window | Command | Reason |
| --- | --- | --- | --- |
| 0s | Win 1 — SITL | arducopter --model JSON ... | Must start first to complete 2-phase boot |
| 12s | Win 2 — Gazebo | gz sim -r iris_ardupilot_runway.sdf | Avoids lock_step deadlock |
| Auto | Win 3 — MAVProxy | mavproxy.py --master=tcp:...:5760 | Connects after SITL is ready |
| Auto | Win 4 — Wireshark | wireshark -i lo -k ... | Captures all MAVLink on UDP 14550 |

## 3.2  Key Issues Resolved During Setup

| Issue | Root Cause | Fix Applied |
| --- | --- | --- |
| link 1 down — MAVProxy exits | -N flag means no-rebuild, not no-mavproxy | Used --no-mavproxy flag correctly |
| Frame: UNSUPPORTED | Old eeprom.bin overriding --defaults | Delete eeprom.bin before each launch |
| Gazebo/SITL deadlock | lock_step=1 with simultaneous start | Start SITL 12s before Gazebo |
| TCP 5760 refused | MAVProxy already owns TCP 5760 | Attack scripts connect via UDP 14550 |
| Signing while armed error | ArduPilot blocks signing setup when armed | Setup signing before arming |

# 4.  Attack Simulations

Two of the four attacks from the paper were successfully simulated and captured in this session. Attacks 3 and 4 are scheduled for the next session.

## 4.1  Attack 1 — Signed Message Clock Manipulation

Paper Reference: Stage 1 — Replication of Ficco et al. (Section IV-C-2a)

### What It Does

A correctly signed MAVLink 2.0 LAND command is crafted with a timestamp set 1 day in the future. When the UAV receives this packet, it validates the signature, advances its internal signing clock by 1 day, and executes the LAND command. This exploits MAVLink Rule 6: receiving a valid signed message updates the local timestamp if the received time is greater.

### How It Works

Connect to MAVProxy's UDP output on port 14550

Enable MAVLink 2.0 signing with key = 'key'

Set signing timestamp to current Unix time + 86400 seconds (1 day)

Send COMMAND_LONG: DO_SET_MODE → LAND (mode 9)

UAV accepts valid signature, clock jumps +1 day, drone lands

### Setup Required in MAVProxy

signing setup key              ← set before arming

signing setup sign_outgoing 1  ← enable signing

mode GUIDED

arm throttle

takeoff 10

### Attack Script

python3 attacks/attack1_signed_cmd.py

### Results

| Metric | Value |
| --- | --- |
| Script execution | Successful |
| Connection method | UDP 14550 (MAVProxy output) |
| Heartbeat received | sysid=1 compid=0 |
| Signing key | 'key' (32-byte padded) |
| Spoofed timestamp | 36006526646826 (2026-05-30 — +1 day) |
| Command sent | COMMAND_LONG: DO_SET_MODE → LAND (mode 9) |
| Capture file | attacks/results/attack1_capture.pcap |
| Total packets captured | 765 |
| Attack packet | Frame 130 — COMMAND_LONG with future timestamp |
| UAV acknowledgement | Frame 131 — COMMAND_ACK received |

### Wireshark Evidence

From attack1_capture.pcap — Frame 130:

Frame 130  |  MAVLink 2.0  |  COMMAND_LONG: MAV_CMD_DO_SET_MODE

Time captured : 2026-05-29 15:53:00.503 IST

Source        : 127.0.0.1:14550  (attack script)

Destination   : 127.0.0.1:57456  (MAVProxy)

Command       : DO_SET_MODE  →  LAND (mode 9)

Signature     : MAVLink 2.0 signed  (SHA-256 truncated)

Timestamp     : 2026-05-30  (+1 day future)

Frame 131  |  COMMAND_ACK: MAV_CMD_DO_SET_MODE  ← UAV responded

### Message Type Breakdown (765 packets)

| Message Type | Count | Meaning |
| --- | --- | --- |
| GLOBAL_POSITION_INT | 29 | Drone GPS position updates |
| ATTITUDE | 28 | Roll, pitch, yaw telemetry |
| SYS_STATUS | 29 | System health status |
| HEARTBEAT | 8 | Drone alive signals |
| COMMAND_LONG | 1 | Our attack packet (Frame 130) |
| COMMAND_ACK | 1 | UAV acknowledgement (Frame 131) |

## 4.2  Attack 2 — GPS Timestamp Spoofing

Paper Reference: Stage 2 — Algorithm 1 (Section IV-C-2b)

### What It Does

Fake GPS_INPUT messages are injected into MAVProxy with a timestamp 10 days in the future. ArduPilot trusts GPS time by default (MAVLink Rule 4) and advances its internal signing clock. No secret key is required — the attacker only needs network access to send UDP packets to MAVProxy's GPS input port.

### How It Works (Algorithm 1)

Load GPSInput module in MAVProxy (listens on UDP 25100)

Set GPS1_TYPE = 14 (accept MAVLink GPS) and SIM_GPS1_DISABLE = 1

Send GPS_INPUT messages at 5 Hz with timestamp = now + 10 days

UAV accepts GPS time → internal clock jumps +10 days

All present-day GCS commands are now rejected as stale

### Setup Required in MAVProxy

param set GPS1_TYPE 14

param set SIM_GPS1_DISABLE 1

module load GPSInput

### Attack Script

python3 attacks/attack2_gps_spoof.py

### Results

| Metric | Value |
| --- | --- |
| Script execution | Successful |
| GPS input port | UDP 127.0.0.1:25100 |
| Clock offset injected | +10 days (864000 seconds) |
| Real time during attack | 2026-05-29 10:30 UTC |
| Spoofed GPS time | 2026-06-08 16:00 UTC |
| Send rate | 5 Hz (5 packets/second) |
| Duration | 30 seconds |
| Total GPS_INPUT packets | 150 packets sent |
| Capture file | attacks/results/attack2_capture.pcap |
| Total packets captured | 1,596 |

### Wireshark Evidence

From attack2_capture.pcap — GPS_INPUT packet (Frame 1):

Frame 1  |  UDP 59503 → 25100  |  GPS_INPUT (MAVLink msg 232)

▼ Payload: GPS_INPUT (232)

time_usec  : 1780914608042893

(2026-06-08 16:00:08.042893 IST)  ← 10 days ahead

gps_id     : 0

fix_type   : 3  (3D Fix)

lat        : -353632610  (-35.363261 deg)

lon        :  1491652370 (149.165237 deg)

alt        : 584.0 m

satellites : 10

time_week_ms: 124208042

### Packet Distribution (1,596 total)

| Traffic Type | Packets | Data | Port |
| --- | --- | --- | --- |
| GPS_INPUT attack packets | 150 | 16 KB | → UDP 25100 |
| Normal MAVLink telemetry | 1,446 | 129 KB | → UDP 14550 |

# 5.  Remaining Attacks (Scheduled Next Session)

| Attack | Name | What It Does | Status |
| --- | --- | --- | --- |
| Attack 3 | Timestamp Overflow DoS | Spoof GPS to Unix 4,234,820,167 — exceeds 48-bit max, all packets permanently rejected | Pending |
| Attack 4 | Replay Attack | Capture signed packet after GPS spoof, replay after clock reset — UAV accepts stale command | Pending |

# 6.  Key Findings

Attack 1 confirmed: A correctly signed MAVLink packet with a future timestamp is accepted by the UAV, causing a forced mode change to LAND — proven by Frame 130 in Wireshark.

Attack 2 confirmed: GPS_INPUT messages with future timestamps successfully advance the UAV's internal signing clock by 10 days — proven by time_usec = 2026-06-08 in Wireshark.

No secret key was needed for Attack 2 — only network access to UDP port 25100.

MAVLink 2.0's timestamp trust model is the root vulnerability — Rules 4 and 6 allow any GPS source to manipulate the signing clock.

SITL environment partially limits signature enforcement (USB bypass), consistent with paper findings that HITL is needed for full cryptographic verification.

# 7.  Project File Structure

drone_security_lab/

├── simulation/

│   └── sitl_basic.py              ← launches all 4 sim windows

├── attacks/

│   ├── attack1_signed_cmd.py       ← Attack 1 script

│   ├── attack2_gps_spoof.py        ← Attack 2 script

│   ├── attack3_overflow_dos.py     ← Attack 3 (next session)

│   ├── attack4_replay.py           ← Attack 4 (next session)

│   └── results/

│       ├── attack1_capture.pcap    ← Wireshark evidence Attack 1

│       └── attack2_capture.pcap    ← Wireshark evidence Attack 2

└── reports/

├── generate_report.py          ← this script

└── attack_report.docx          ← output report

# 8.  Next Steps

Simulate Attack 3 — Timestamp Overflow DoS: spoof GPS to Unix 4234820164, push past 48-bit limit, observe permanent packet rejection.

Simulate Attack 4 — Replay Attack: capture signed packet during spoofed session, restart UAV without key rotation, replay to confirm acceptance.

Document all 4 attacks in a combined Wireshark analysis.

Write countermeasures section based on paper Section IV-F.