Internship Daily Report

June 2, 2026

Goal: Align all attack scripts with the reference implementation (JSON GPS injection, manual SHA-256 signing), add Gazebo-visible position spoofing to Attack 2, implement Attack 3B (timestamp underflow), build Wireshark Lua dissectors for all attacks, and complete a full live Attack 4 replay demo in Gazebo.

Stack: ArduPilot SITL v4.8.0-dev · Gazebo Harmonic · MAVProxy 1.8.74 · Wireshark 3.6.2 + Lua 5.1 · Python 3.10.12 · pymavlink 2.4.49

## Work Completed

### Attack Scripts Aligned with Reference Implementation

Attack 1 — switched from pymavlink signing API to manual SHA-256 signing (matching reference custom-input.py).

Key is now hashed with SHA256 before use — same method as MAVProxy internally. Old code padded raw bytes with zeros (wrong).

Packet built with struct.pack + mavcrc.x25crc_slow + hashlib.sha256. Sent with conn.write(raw_bytes) not command_long_send().

Attacks 2 & 3 — GPS injection switched from pymavlink binary MAVLink to JSON over raw UDP socket.

MAVProxy GPSInput module on UDP 25100 expects JSON, not binary MAVLink. Reference simulate-gps.py confirms this format.

Removed pymavlink dependency for GPS injection — just socket.sendto(json.dumps(data).encode(), (host, 25100)).

### Attack 2 — Position Spoofing Added (Gazebo Visible)

Previous Attack 2 only spoofed the timestamp — drone had no reason to move, no visual effect in Gazebo.

Added SPOOF_LAT_DRIFT = -0.0018 degrees: reported GPS position drifts 200m south over 30 seconds.

Drone in GUIDED mode compensates for the fake southward drift by flying NORTH — visible in Gazebo.

Two effects demonstrated simultaneously: clock +10 days (invisible) + drone moves 200m north (visible).

Verified: MAVProxy 'time' command shows 10 days in future. 'mode LAND' command rejected after attack.

### Attack 3B — Timestamp Underflow DoS (New Attack)

New attack: inject GPS_INPUT with time_usec representing Jan 1 2010 — 5 years BEFORE the MAVLink epoch (Jan 1 2015).

MAVLink signing timestamp = (unix - epoch) * 100000 = negative value = -15,776,640,000,000.

Cast to uint64: wraps to 18,446,728,297,069,551,616 — larger than Attack 3's overflow value.

Two possible effects: Effect A = DoS (same as Attack 3 via uint64 wrap) / Effect B = clock reset to 0 (accepts all future packets).

Script: attacks/attack3b_underflow_dos.py — sends 10 burst packets, then verifies effect by sending a real signed command.

Verification: if UAV responds = clock reset (Effect B). If silent = DoS (Effect A).

### Wireshark Lua Dissectors — All Attacks Decoded in GUI

gps_spoof.lua: decodes all GPS_INPUT packets on UDP 25100 into 3 attack zones.

Zone 0 (Attack 3B): time_usec < Jan 2015 — shows signed MAVLink TS, uint64 wrapped value, 48-bit masked value.

Zone 2 (Attack 2): time_usec in 2026 range — shows +10 day gap, lat drift in metres.

Zone 3 (Attack 3): time_usec > 3e15 — shows 78-year gap, MAVLink TS vs 2^48-1, DoS effect.

attack4_replay.lua: post-dissector on all packets, annotates COMMAND_LONG (76) and COMMAND_ACK (77).

Shows: signature date, hours in future, command decoded as LAND, ACK result=0 (ACCEPTED).

Fixed: register_postdissector(p, true) — allfields=true makes fields indexable for display filters.

Fixed Lua uint64 overflow: UINT64_MAX cannot be stored exactly in Lua double (max exact int = 2^53). Replaced with pre-computed string constants.

Fixed decode_as_entries conflict: port 25100 was locked to MAVLINK_PROTO from a previous session, overriding our dissector.

### Complete Attack 4 Replay Demo in Gazebo

Full end-to-end Attack 4 sequence documented and tested in Gazebo.

Drone flies to waypoint (-35.3619, 149.1652) at 20m AGL — visibly airborne in Gazebo.

attack4_replay.py replays signed COMMAND_LONG from attack1_capture.pcap byte-for-byte.

UAV accepts the replay (COMMAND_ACK result=0), switches to LAND mode, drone descends in Gazebo.

Key condition: SITL rebooted (clock reset) but key NOT rotated — old future timestamp still valid.

Timestamp validity check script added — warns if attack1 capture timestamp has expired (>24h old).

### Live Wireshark Capture Setup

Documented live capture on loopback (lo) interface with capture filter: udp port 25100 or udp port 14550.

Dissectors apply automatically to live traffic — same decoded fields appear in real-time during attacks.

Confirmed: decode_as_entries persists across sessions — port 25100 → atk2gps, port 14550 → mavlink_proto.

## Issues Faced & Resolutions

Signing key derivation mismatch — old code used raw bytes padded to 32 bytes; MAVProxy uses sha256(key_string).

Resolution: replaced ljust(32, b'\x00') with hashlib.sha256(key_str.encode()).digest().

GPS_INPUT format — old code sent binary MAVLink packets; GPSInput module expects JSON.

Resolution: replaced mavutil.mavlink_connection + gps_input_send() with raw UDP socket + json.dumps().

No visual effect in Gazebo for Attack 2 — timestamp spoofing alone doesn't move the drone.

Resolution: added gradual lat drift (-0.0018 degrees over 30s) — drone compensates northward, fully visible.

Lua uint64 overflow — UINT64_MAX = 2^64-1 cannot be stored exactly in Lua double precision (max 2^53).

Resolution: pre-computed Attack 3B wrapped values stored as string constants. Only 48-bit masked value (< 2^53) kept as number.

Wireshark filter atk4replay.type returned 0 results — post-dissector fields not indexed by default.

Resolution: changed register_postdissector(p) to register_postdissector(p, true) to enable field indexing.

Port 25100 locked to MAVLINK_PROTO in decode_as_entries from a previous session.

Resolution: edited ~/.config/wireshark/decode_as_entries to replace MAVLINK_PROTO with atk2gps.

Drone auto-disarming during flight — arm throttle rejected pre-arm checks (GPS/EKF).

Resolution: use arm throttle force to bypass pre-arm checks for SITL testing.

SIM_GPS1_DISABLE parameter not found — not available in JSON SITL model.

Resolution: parameter not needed with JSON model — GPS1_TYPE=14 is sufficient for GPSInput to take priority.

## Attack Results Summary

| Attack | Method | Visible in Gazebo | Wireshark Protocol | Pcap |
| --- | --- | --- | --- | --- |
| 1 — Clock Manip. | Manual SHA-256 signed CMD | Drone lands mid-flight | ATK4_REPLAY | attack1_capture.pcap |
| 2 — GPS Spoof | JSON GPS +10d + lat drift | Drone flies 200m north | ATK2_GPS_SPOOF | attack2_capture.pcap |
| 3 — Overflow DoS | JSON GPS at year 2104 | Drone ignores all commands | ATK3_OVERFLOW | attack3_capture.pcap |
| 3B — Underflow DoS | JSON GPS at year 2010 | DoS or clock reset (impl. dep.) | ATK0_UNDERFLOW | attack3b_capture.pcap |
| 4 — Replay | Raw bytes from Attack 1 | Drone lands again after reboot | ATK4_ACCEPTED | attack4_capture.pcap |

## Wireshark Filter Reference

| Filter | Shows | Attack |
| --- | --- | --- |
| udp.dstport == 25100 | All GPS injection packets | 2, 3, 3B |
| mavlink_proto.msgid == 76 ¦¦ mavlink_proto.msgid == 77 | COMMAND_LONG + COMMAND_ACK | 1, 4 |
| atk4replay.type | Only replay & ACK packets | 1, 4 |
| frame.protocols contains "atk4replay" | All packets our dissector touched | 1, 4 |
| atk2gps.gap | Packets with 10-day clock gap | 2 |

## Next Steps

Run all 5 attacks in a single session with live Wireshark open — capture each pcap fresh.

Add Wireshark coloring rules so each attack protocol shows in a distinct colour.

Update the full technical report (generate_full_report_v2.py) to include Attack 3B and all dissector details.

Document the arm throttle force workaround and SIM_GPS1_DISABLE findings in the project README.