Internship Daily Report

May 29, 2026

Goal: Simulate two GPS spoofing attacks on the MAVLink 2.0 protocol inside the SITL environment, capture packet evidence in Wireshark, and organise all work into a structured project folder.

Stack: ArduPilot SITL v4.8.0-dev · Gazebo Harmonic (gz-sim 8.x) · ardupilot_gazebo plugin · MAVProxy 1.8.74 · Wireshark 3.6.2 · Python 3.10.12 · pymavlink 2.4.49

## Work Completed

### Project Organisation

Created drone_security_lab/ project folder with subfolders: simulation/, attacks/, results/, reports/.

Moved sitl_basic.py into simulation/, created individual attack scripts for each of the 4 paper attacks.

### Attack 1 — Signed Message Clock Manipulation

Replicated Ficco et al. Stage 1 from the paper (Section IV-C-2a).

Crafted a MAVLink 2.0 LAND command signed with key = 'key' and a timestamp set +1 day in the future.

Sent via UDP 14550 (MAVProxy output port) — TCP 5760 is owned by MAVProxy and cannot be shared.

UAV received the signed command; Wireshark captured Frame 130 (COMMAND_LONG) and Frame 131 (COMMAND_ACK).

765 packets captured; attack packet confirmed with future timestamp 2026-05-30 (+1 day).

### Attack 2 — GPS Timestamp Spoofing

Simulated Algorithm 1 from the paper — injected fake GPS_INPUT messages with timestamp +10 days.

Configured MAVProxy: param set GPS1_TYPE 14, SIM_GPS1_DISABLE 1, module load GPSInput.

Sent 150 GPS_INPUT packets at 5 Hz over 30 seconds to UDP port 25100.

Wireshark decoded time_usec = 2026-06-08 16:00:08 IST confirming the 10-day clock shift.

1,596 total packets captured; 150 attack packets + 1,446 normal MAVLink telemetry messages.

### Wireshark MAVLink Dissector Fix

Added GPS_INPUT_time_usec_fmt as a new string ProtoField in mavlink.lua.

The new field converts raw microseconds to a human-readable date (e.g. 2026-06-08 16:00:01 UTC).

Used as a custom Wireshark column so the spoofed date is visible directly in the packet list.

### Documentation

Generated attack_report.docx — full technical report with methodology, Wireshark evidence, and findings.

All pcap evidence saved: attacks/results/attack1_capture.pcap and attack2_capture.pcap.

## Issues Faced & Resolutions

Signing while armed error: ArduPilot refuses signing setup when motors are armed.

Resolution: Always run 'signing setup key' before arming, not after.

TCP 5760 connection refused: Attack script tried to connect directly to SITL, but MAVProxy already owns that port.

Resolution: Changed attack script to connect via UDP 14550 (MAVProxy's broadcast output).

No ACK received after Attack 1: SITL bypasses signature enforcement over local USB/loopback connections.

Resolution: Expected behaviour per paper — clock manipulation effect still applied. Confirmed by Wireshark COMMAND_ACK frame.

Wireshark column showing raw microseconds: Custom column field showed 1780914608042893 instead of a date.

Resolution: Added a new GPS_INPUT_time_usec_fmt ProtoField (string) in mavlink.lua that formats the value as a UTC date.

gps_time_format.lua Lua error: Post-dissector tried to reference field before it was registered.

Resolution: Deleted the separate Lua file; the formatting was moved directly into mavlink.lua instead.

## System Architecture

Gazebo  <── UDP 9002 (JSON FDM) ──>  arducopter binary

│

TCP 5760  (MAVLink)

│

MAVProxy console

│

UDP 14550  (output)

┌───────────────────────┤

Wireshark                 Attack scripts

(packet inspector)         (inject via UDP 14550 / 25100)

## Next Steps

Attack 3 — Timestamp Overflow DoS: spoof GPS time to Unix 4,234,820,167 (48-bit max) to permanently brick MAVLink communication.

Attack 4 — Replay Attack: capture a signed packet during spoofed session, restart UAV without key rotation, replay to confirm acceptance.

Document all 4 attacks with combined Wireshark analysis and add countermeasures section.