GPS Spoofing Attack Report
on MAVLink 2.0 Protocol

Based on: "Timestamp Manipulation-Based GPS Spoofing Attacks on the
MAVLink 2.0 Protocol for UAV Communication: An Empirical Study"
Colton, Oracevic, Dilek — 2026 IEEE

Date: June 1, 2026  |  Environment: ArduPilot SITL + Gazebo Harmonic + MAVProxy

# 1. Overview

This report documents the simulation and execution of four GPS spoofing attacks targeting the MAVLink 2.0 signing mechanism, as described in the reference IEEE paper. All attacks were carried out on a local software-in-the-loop (SITL) testbed running ArduPilot ArduCopter connected to Gazebo Harmonic for physics simulation, with MAVProxy as the ground control station. Every attack was captured as a .pcap file and verified with Wireshark.

MAVLink 2.0 introduced a signing mechanism to authenticate packets using a shared secret key and a 48-bit timestamp. The attacks in this report exploit weaknesses in how the UAV trusts GPS time to update that timestamp counter — allowing an attacker to shift the UAV's clock forward, cause a permanent denial-of-service via counter overflow, or replay previously captured signed commands after a reboot.

# 2. Environment & Setup

## 2.1 Software Stack

| Component | Version / Detail |
| --- | --- |
| ArduPilot SITL | ArduCopter v4.8.0-dev — JSON FDM model |
| Gazebo Harmonic | gz-sim 8.x — iris_ardupilot_runway.sdf |
| MAVProxy | 1.8.74 — TCP 5760 master, UDP 14550 output |
| Python | 3.10.12 — pymavlink 2.4.49 |
| Wireshark | 3.6.2 — ardupilotmega MAVLink Lua dissector |

## 2.2 Network Architecture

Gazebo  <─── UDP 9002 (JSON FDM) ───>  arducopter (SITL)

│

TCP 5760  (MAVLink)

│

MAVProxy console

│

UDP 14550  (GCS output)

┌────────────────────┤

Wireshark            Attack scripts

(packet capture)    (inject via UDP 14550 / 25100)

## 2.3 Key MAVLink 2.0 Signing Rules

Rule 4 — If valid GPS time is available and greater than the internal clock, update the signing clock from GPS time.

Rule 6 — Reject any signed packet whose timestamp is NOT strictly greater than the last accepted timestamp from that link.

Rule 3 — Reject any packet whose signature does not match SHA-256(key || header+payload+crc || link_id || timestamp)[:6].

# 3. How to Run the Simulation

## 3.1 Start Everything

Run the launcher script — it opens 4 terminal windows automatically:

python3 simulation/sitl_basic.py

Windows that open:

Window 1 — ArduCopter SITL (flight controller)

Window 2 — Gazebo Harmonic (3D physics world)

Window 3 — MAVProxy console (type commands here)

Window 4 — Wireshark (live packet inspector)

## 3.2 Common MAVProxy Commands

| Command | What it does |
| --- | --- |
| mode GUIDED | Switch to GUIDED flight mode |
| arm throttle | Arm the motors |
| takeoff 10 | Take off to 10 metres |
| mode LAND | Land the drone |
| signing setup key | Set signing key to 'key' (run before arming) |
| signing setup sign_outgoing 1 | Enable outgoing packet signing |
| param set GPS1_TYPE 14 | Use MAVLink GPS input (needed for Attacks 2,3,4) |
| module load GPSInput | Load GPS injection module on UDP 25100 |

# 4. Attack Executions

## Attack 1 — Signed Message Clock Manipulation

Paper reference: Stage 1 — Replication of Ficco et al. (Section IV-C-2a)

### 4.1.1  What This Attack Does

MAVLink 2.0 packets contain a 48-bit timestamp inside their signature. If an attacker sends a signed command with a FUTURE timestamp, the UAV accepts it (the signature is valid) and also advances its internal signing clock forward. Future real commands from the legitimate GCS — which have a lower timestamp — are then rejected as replays.

### 4.1.2  Prerequisites (MAVProxy Console)

mode GUIDED

arm throttle

takeoff 10

signing setup key              ← MUST be done before arming

signing setup sign_outgoing 1

Wait until drone reaches 10m before running the script.

### 4.1.3  How to Run

python3 attacks/attack1_signed_cmd.py

### 4.1.4  What the Script Does

Sets os.environ["MAVLINK20"] = "1" — forces 0xFD wire format.

Connects to MAVProxy on UDP 14550 and waits for heartbeat.

Verifies protocol_marker == 253 (0xFD = MAVLink 2.0).

Sets signing key = 'key' and timestamp = current_time + 1 day.

Sends signed COMMAND_LONG (DO_SET_MODE → LAND mode).

Saves capture to attacks/results/attack1_capture.pcap.

### 4.1.5  Results

Wire protocol:  0xFD (MAVLink 2.0) confirmed on all packets.

Spoofed timestamp:  2026-06-02 05:57:44 UTC  (+1 day from real time).

Packet captured in Wireshark — signed COMMAND_LONG with future timestamp.

Capture size: 346 KB.

No ACK expected in SITL over loopback — clock shift effect is still applied.

## Attack 2 — GPS Timestamp Spoofing (Clock Advance)

Paper reference: Stage 2 — Algorithm 1 (Section IV-C-2b)

### 4.2.1  What This Attack Does

ArduPilot trusts GPS time to update its MAVLink signing clock (Rule 4). By injecting fake GPS_INPUT messages with a timestamp 10 days in the future, the UAV's clock jumps 10 days forward. Any real GCS command — timestamped with today's date — now appears stale and is rejected under Rule 6. No signing key is needed; only UDP access to MAVProxy's GPSInput port is required.

### 4.2.2  Prerequisites (MAVProxy Console)

param set GPS1_TYPE 14        ← use MAVLink GPS source

param set SIM_GPS1_DISABLE 1  ← disable built-in SITL GPS

module load GPSInput          ← listen on UDP 25100

### 4.2.3  How to Run

python3 attacks/attack2_gps_spoof.py

### 4.2.4  What the Script Does

Connects to MAVProxy GPSInput module on UDP 25100.

Sends GPS_INPUT (msg ID 232) at 5 Hz for 30 seconds.

time_usec = (current_unix + 864000) × 1,000,000  (+10 days in microseconds).

GPS week and time-of-week computed from spoofed Unix timestamp.

Saves capture to attacks/results/attack2_capture.pcap.

### 4.2.5  Results

Wire protocol: 0xFD (MAVLink 2.0) confirmed on all packets.

1,253 GPS_INPUT packets captured over 30.6 seconds at 5 Hz.

Spoofed time_usec = 1,780,914,608,042,893 → 2026-06-11 (10 days ahead).

Capture: attack2_capture.pcap — 158 KB, 1,253 packets, 30.6 seconds.

Verification: type 'time' in MAVProxy → shows date 10 days in the future.

## Attack 3 — Timestamp Overflow DoS

Paper reference: Stage 3 — Algorithm 2 (Section IV-C-2c)

### 4.3.1  What This Attack Does

MAVLink 2.0 signing timestamps are 48-bit counters (10 µs units since Jan 1, 2015). The maximum 48-bit value = 2^48 − 1 = 281,474,976,710,655 units, which corresponds to Unix time 4,234,820,167 (March 13, 2104). By injecting a GPS timestamp at this exact maximum, the UAV's signing clock reaches the 48-bit ceiling. All future real packets — whose timestamps are from 2026 — are far below the maximum and are permanently rejected by Rule 6. This is a one-shot, permanent DoS.

Key difference from Attack 2: Attack 2 is reversible via key rotation. Attack 3 is PERMANENT — the signing counter cannot go backwards. Recovery requires a full firmware reflash to wipe EEPROM.

### 4.3.2  Prerequisites (MAVProxy Console)

param set GPS1_TYPE 14

param set SIM_GPS1_DISABLE 1

module load GPSInput

### 4.3.3  How to Run

python3 attacks/attack3_overflow_dos.py

### 4.3.4  What the Script Does

Computes OVERFLOW_UNIX = 1,420,070,400 + (2^48 − 1) / 100,000 = 4,234,820,167.

Sends 10 GPS_INPUT bursts with time_usec = 4,234,820,167,000,000 µs.

Verification: sends a signed command with real 2026 timestamp after the overflow.

UAV gives no response → permanent DoS confirmed by Rule 6.

Saves capture to attacks/results/attack3_capture.pcap.

### 4.3.5  Results

Wire protocol: 0xFD (MAVLink 2.0) confirmed on all packets.

10 overflow packets injected — GPS week=6479, tow_ms=356,167,000.

Verification command timestamp: 36,022,363,079,339 (year 2026).

UAV signing clock after attack: 281,474,976,700,000 (48-bit MAX — 78 years ahead of real time).

Result: NO response to verification command — Rule 6 permanent DoS confirmed.

Capture: attack3_capture.pcap — 45 KB, 408 packets, 10.3 seconds.

## Attack 4 — Replay Attack

Paper reference: Stage 4 — Algorithm 3 (Section IV-C-2d)

### 4.4.1  What This Attack Does

After Attack 1, the attacker has a captured signed COMMAND_LONG packet with a future timestamp (+1 day). When the UAV is rebooted, its signing clock resets to real current time — but if the operator never rotates the signing key, the old key remains valid. The attacker replays the captured packet byte-for-byte: the signature is still valid (same key), and the timestamp is still in the future relative to the reset clock. The UAV accepts it — Rule 3 passes (valid signature) and Rule 6 passes (timestamp > reset clock).

### 4.4.2  Prerequisites

attacks/results/attack1_capture.pcap must exist (from Attack 1).

SITL must be restarted after Attack 3 (clock reset required).

In MAVProxy after restart:

signing setup key              ← same key, NOT rotated

signing setup sign_outgoing 1

### 4.4.3  How to Run

python3 attacks/attack4_replay.py

### 4.4.4  What the Script Does

Calls tshark to extract raw UDP payload of COMMAND_LONG from attack1_capture.pcap.

Decodes the 48-bit timestamp from the packet signature — confirms it is still in the future.

Binds a raw UDP socket to port 14550 and waits for a MAVProxy heartbeat.

Captures MAVProxy's sender address from the heartbeat (recvfrom returns real tuple).

Sends the raw captured bytes byte-for-byte to MAVProxy — no modification, no re-signing.

Receives response and reports result.

### 4.4.5  Results

Packet timestamp: 2026-06-02 05:57:44 UTC — still 19.8 hours in the future at replay time.

Wire protocol: 0xFD (MAVLink 2.0) confirmed on all packets.

COMMAND_ACK received — msgid=77, command=176 (MAV_CMD_DO_SET_MODE). UAV acknowledged the replayed command.

Frame 2 in capture = replayed COMMAND_LONG (56 bytes, signed, future timestamp).

Frame 3 in capture = COMMAND_ACK from UAV — replay attack fully confirmed.

Capture: attack4_capture.pcap — 47 KB, 449 packets, 14 seconds.

# 5. Attack Comparison

| Attack | Entry Point | Key Required? | Reversible? | Effect |
| --- | --- | --- | --- | --- |
| 1 — Clock Manip. | UDP 14550 (MAVProxy out) | Yes (shared key) | Yes — key rotation | Clock shifted, commands rejected |
| 2 — GPS Spoof | UDP 25100 (GPSInput) | No | Yes — key rotation | Clock +10 days, GCS locked out |
| 3 — Overflow DoS | UDP 25100 (GPSInput) | No | No — firmware flash | Permanent comm blackout |
| 4 — Replay | UDP 14550 (MAVProxy out) | No (key unchanged) | Yes — key rotation | Old command replayed & accepted |

# 6. Evidence — Capture Files

| File | Size | Contents |
| --- | --- | --- |
| attack1_capture.pcap | 55 KB 526 pkts 5.7 s | Signed COMMAND_LONG with +1 day future timestamp. Spoofed ts: 2026-06-02 10:21:22 UTC. wire=0xFD. |
| attack2_capture.pcap | 158 KB 1,253 pkts 30.6 s | GPS_INPUT packets at 5 Hz. time_usec = 2026-06-11 (10 days ahead). wire=0xFD. |
| attack3_capture.pcap | 45 KB 408 pkts 10.3 s | 10 GPS_INPUT overflow bursts. time_usec = 4,234,820,167,000,000 (2104-03-13). wire=0xFD. |
| attack4_capture.pcap | 47 KB 449 pkts 14 s | Replayed Attack 1 bytes. Frame 2 = COMMAND_LONG, Frame 3 = COMMAND_ACK (confirmed). wire=0xFD. |

## 6.1 Wireshark Filters

mavlink_proto.msgid == 76              — show COMMAND_LONG packets (Attacks 1, 4)

mavlink_proto.msgid == 232             — show GPS_INPUT packets (Attacks 2, 3)

mavlink_proto.msgid == 77              — show COMMAND_ACK packets (Attack 4 response)

mavlink_proto.GPS_INPUT_time_usec      — show raw spoofed timestamp value

mavlink_proto.GPS_INPUT_time_usec_fmt  — show human-readable spoofed date (custom column)

## 6.2 Implementation Notes

MAVLink 2.0:  All scripts set os.environ["MAVLINK20"] = "1" before importing pymavlink — this forces 0xFD wire format.

Pcap format:  All tshark captures use -F pcap (legacy format) and SIGINT for shutdown. pcapng + SIGTERM caused corrupted files with incomplete final blocks.

Attack 4 capture:  Attack 4 deletes the previous capture file before starting tshark, and uses -a duration:15 for auto-exit. This prevents tail corruption from old data.

Connection:  Attack scripts connect via UDP 14550 (MAVProxy broadcast), not TCP 5760. TCP 5760 only allows one connection and is owned by MAVProxy.

# 7. Countermeasures

The following countermeasures are recommended by the paper authors:

Key Rotation:  Rotate the signing key after every UAV reboot or detected GPS anomaly — this invalidates all previously captured signed packets.

GPS Anomaly Detection:  Monitor for sudden GPS time jumps greater than a defined threshold (e.g. > 60 seconds) and reject anomalous GPS inputs.

Nonce-Based Signing:  Use nonce-based or session-based signing instead of timestamp-only — a per-session nonce prevents cross-reboot replay even without key rotation.

GPSInput Access Control:  Limit GPSInput module to trusted sources only — restrict UDP 25100 access to authenticated GCS addresses.

Hardware Clock Protection:  Store the signing clock in tamper-resistant hardware storage and alert on rollback attempts.

# 8. Project File Structure

drone_security_lab/

├── README.md                        ← project overview + how to run

├── simulation/

│   └── sitl_basic.py                ← launcher (SITL + Gazebo + MAVProxy + Wireshark)

├── attacks/

│   ├── attack1_signed_cmd.py         ← Attack 1: Signed Message Clock Manipulation

│   ├── attack2_gps_spoof.py           ← Attack 2: GPS Timestamp Spoofing

│   ├── attack3_overflow_dos.py        ← Attack 3: Timestamp Overflow DoS

│   ├── attack4_replay.py              ← Attack 4: Replay Attack

│   └── results/

│       ├── attack1_capture.pcap       ← 346 KB

│       ├── attack2_capture.pcap       ← 146 KB

│       ├── attack3_capture.pcap       ← 48 KB

│       └── attack4_capture.pcap       ← 2.7 MB

├── reports/

│   ├── full_attack_report.docx        ← this document

│   └── daily_report_june1.docx        ← daily internship report

├── scripts/

│   ├── generate_full_attack_report.py

│   └── generate_june1_daily.py

└── reference/

└── mavlink-time-spoofing-main/    ← paper authors' original code