Internship Daily Report

# June 8, 2026 — Day 5

Goal: Live detection of all five MAVLink timestamp attacks using the Step 2 statistical rule engine

Stack: ArduPilot SITL v4.8.0-dev  |  MAVProxy 1.8.74  |  Python 3.10  |  pymavlink 2.4.49

## 1. SITL Environment Fix

Root cause: ArduPilot was found in the system Trash (/home/jatin/.local/share/Trash/files/ardupilot). Restored with mv to ~/ardupilot — no recompile needed.

GPS1_TYPE bug: mav.parm persisted GPS1_TYPE=14 (MAVLink GPS_INPUT) across sessions. With no GPSInput JSON stream at boot, SITL reported 'GPS 1: Bad fix' and refused to arm. Fix: delete mav.parm before each session so SITL uses compiled-in defaults (GPS1_TYPE=1, auto).

Battery failsafe: SITL battery simulation drains over multi-attack sessions. BATT_LOW_VOLT and BATT_CRT_VOLT set to 0 at session start. SIM_BATT_VOLTAGE reset to 12.6 V when needed.

## 2. Detector Monitor Port Architecture

The detector (Step 2) listens on UDP 14550 (MAVLink) and UDP 25100 (GPS_INPUT) using SO_REUSEPORT to share ports with MAVProxy. However, attack scripts for Attacks 1 and 4 send MAVLink packets directly to MAVProxy's ephemeral source port — not to 14550 — so the detector never saw them.

Fix: two dedicated monitor ports were added:

Port 14551 — MAVLink monitor  (Attacks 1, 4 mirror here)
Port 25101 — GPS monitor       (Attacks 2, 3, 3B mirror here)

Each attack script sends the attack packet to its real destination (MAVProxy) AND a copy to the monitor port. The detector listens exclusively on the monitor ports for these attack types, eliminating the SO_REUSEPORT packet-distribution conflict.

## 3. Attack Runs and Detection Results

Attack 1 — Signed Command Injection → ATK1_FUTURE_TS

Sent COMMAND_LONG (LAND) signed with SHA-256, timestamp +24 h. Detector flagged ts_gap = +86,400 s >> 3,600 s threshold. Drone executed LAND.

Attack 2 — GPS Timestamp + Position Spoof → ATK2_GPS_SPOOF + ATK3B_BURST

Injected GPS_INPUT JSON with time_usec +10 days, position drifting 200 m south over 30 s. gps_sys_delta = +864,000 s detected. Burst rule also fired (5 Hz > threshold).

Attack 3 — Timestamp Overflow DoS → ATK3_OVERFLOW

GPS time_usec set to year 2104 (4,234,820,167,000,000 µs). Detector confirmed GPS date +78 years. Signing clock advanced to 2^48-1 — drone rejected all subsequent commands. SITL restart required before Attack 3B.

Attack 3B — Timestamp Underflow DoS → ATK3B_UNDERFLOW

GPS time_usec set to year 2010 (pre-epoch, before Jan 1 2015). Detector confirmed GPS date < MAVLINK_EPOCH. Signing timestamp computation yields negative value → uint64 wrap to ~2^64, causing clock reset or rejection.

Attack 4 — Replay Attack → ATK4_REPLAY + ATK1_FUTURE_TS

After SITL reboot (clock reset, key unchanged), Attack 1 capture was replayed verbatim. Two alerts fired simultaneously: duplicate SHA-256 hash (ATK4_REPLAY) and future timestamp still valid after clock reset (ATK1_FUTURE_TS). Confirms MAVLink 2.0 lacks replay protection when keys are not rotated post-incident.

## 4. Alert Statistics (June 8 Session)

| Alert Tag | Count | Notes |
| --- | --- | --- |
| ATK1_FUTURE_TS | 2 | Attack 1 send + Attack 4 replay |
| ATK2_GPS_SPOOF | 150 | 5 Hz × 30 s = 150 packets |
| ATK3_OVERFLOW | 10 | Burst of 10 overflow packets |
| ATK3B_UNDERFLOW | 10 | Burst of 10 underflow packets |
| ATK3B_BURST | 155 | Burst detector (rate > 5 pkts/2 s) |
| ATK4_REPLAY | 18 | Replay loop × 18 iterations |
| Total | 345 | All from statistical rules — no ML yet |

## 5. Next Steps

Step 3 — train_baseline.py: capture 5 minutes of normal SITL traffic (no attacks), extract feature vectors, train Isolation Forest with contamination=0.05, save model as detection/models/baseline.pkl.

Step 4 — Integrate ML layer: load baseline.pkl in detector.py, score each packet, emit ATK_ML_ANOMALY alert when Isolation Forest score < threshold alongside the statistical rules.

Step 5 — Final validation: run all 5 attacks against the two-layer detector, confirm statistical rules + ML both fire, measure false positive rate on 5 min clean traffic.

Upload final pcap captures, alerts.log, and baseline.pkl to Google Drive.