MAVLink v2 IDS — Complete Run Guide

Step-by-step instructions with code snippets
Attack Detection + Secure Timestamp Defense

1.  What We Built

We built a complete MAVLink v2 security system consisting of two parts: an Intrusion Detection System (IDS) that detects four real attacks in real time, and a Secure Timestamp Defense proxy that blocks those attacks before they reach the drone.

| Part | What it does | File |
| --- | --- | --- |
| IDS Detector | Detects 4 attacks using rules + ML | realtime_detector_v4.py |
| ML Model | Random Forest, F1=1.0 on 5-fold CV | models/rf_multiclass_v2.pkl |
| Attack Scripts | Simulate each attack in SITL | attacks/attack1-4.py |
| Defense Proxy | Blocks attacks before they reach the drone | defense/secure_timestamp_defense.py |
| SITL Launcher | Starts ArduCopter + Gazebo + MAVProxy | simulation/sitl_basic.py |
| GPS Simulator | Feeds GPS time to detector and defense | simulate_normal_gps.py |

2.  System Overview

Data flow during normal operation:

[ArduCopter SITL] ──TCP 5760──► [MAVProxy] ──UDP 14550──► [IDS Detector] ▲ │ │ Gazebo 3D UDP 14551 (mirror) [ML Model] (UDP 9002) [Alert Log] [GPS Simulator] ──UDP 25101──► [IDS Detector] (GPS cross-check) During Defense Demo: [Attacker] ──UDP 14549──► [Defense Proxy] ──UDP 14550──► [Detector/SITL] │ (drops bad packets)

| Port | Purpose |
| --- | --- |
| TCP 5760 | ArduCopter SITL ↔ MAVProxy |
| UDP 14550 | MAVProxy output → IDS Detector |
| UDP 14551 | Mirror for signed attack packets (custom-input.py) |
| UDP 25101 | GPS time feed (JSON) → Detector and Defense proxy |
| UDP 14549 | Defense proxy input (attacker sends here) |

3.  Part 1 — Running the IDS

You need 4 terminals for this part.

Terminal 1 — Launch SITL + Gazebo + MAVProxy

Step 1  Start the simulation

cd /home/jatin/Documents/drone_security_lab/simulation python3 sitl_basic.py

Note: This opens 4 windows automatically: SITL, Gazebo, MAVProxy, Wireshark. Wait ~60 seconds until MAVProxy shows 'MAV>' before continuing.

Wait until you see in the MAVProxy window:

Mode: STABILIZE Heartbeat from APM MAV>

[ SCREENSHOT 1: All 4 windows open — SITL + Gazebo + MAVProxy + Wireshark ]

MAVProxy Window — Setup signing and fly

Step 2  Enable MAVLink signing

signing setup key signing setup sign_outgoing 1

Step 3  Arm and take off

mode GUIDED arm throttle takeoff 10

[ SCREENSHOT 2: Drone taking off in Gazebo ]

Terminal 2 — Start GPS feed

Step 4  Start the GPS simulator

cd /home/jatin/Documents/drone_security_lab python3 simulate_normal_gps.py

Note: Leave this running. It feeds GPS timestamps to the detector on UDP 25101.

Terminal 3 — Start the IDS detector

Step 5  Start the detector

cd /home/jatin/mavlink-detection python3 realtime_detector_v4.py

Note: The detector listens on UDP 14550 (MAVLink), 14551 (signed mirror), 25101 (GPS).

You should see:

[*] Listening on UDP 14550 [*] Mirror socket : UDP 14551 [*] GPS mirror : UDP 25101 [*] Model loaded : models/rf_multiclass_v2.pkl [NORMAL] seq=1 sysid=1 ...

[ SCREENSHOT 3: Detector running — showing NORMAL packets ]

4.  Part 2 — Running the Attacks

Run each attack from a new terminal. The detector (Terminal 3) will alert.

Attack 1 — Timestamp Injection

Sends a signed MAVLink packet with timestamp set 1 day in the future. The drone's signing clock advances, enabling forced landing.

Step 6  Run Attack 1

cd /home/jatin/Documents/drone_security_lab/attacks python3 attack1_signed_cmd.py

Detector output:

[ATTACK-1 Timestamp Injection] sig_ts +86400s ahead of wall clock Rule fired: SIG_FUTURE (offset=86400.0s > threshold 60s) Confidence: RULE 100%

[ SCREENSHOT 4: Detector alerting Attack 1 ]

Attack 2 — GPS Spoofing

Injects fake GPS coordinates via the GPS input port. GPS time mismatches by 300–1,000,000,000 seconds triggers the rule.

Step 7  Run Attack 2

cd /home/jatin/Documents/drone_security_lab/attacks python3 attack2_gps_spoof.py

Detector output:

[ATTACK-2 GPS Spoofing] GPS mismatch = 450.3s (threshold: 300s–1B s) Rule fired: GPS_MISMATCH Confidence: RULE 100%

[ SCREENSHOT 5: Detector alerting Attack 2 ]

Attack 3 — Overflow / DoS

Sends GPS data with an astronomically large timestamp (> 1 billion seconds offset). This causes a timestamp overflow and can freeze the flight controller.

Step 8  Run Attack 3

cd /home/jatin/Documents/drone_security_lab/attacks python3 attack3_overflow_dos.py

Detector output:

[ATTACK-3 Overflow / DoS] GPS mismatch = 2,814,749,767s (> 1B threshold) Rule fired: GPS_OVERFLOW Confidence: RULE 100%

[ SCREENSHOT 6: Detector alerting Attack 3 ]

Attack 4 — Replay Attack

Replays a previously captured future-signed packet after SITL reset. The key was never rotated so the old signature is still valid.

Step 9  Run Attack 4

cd /home/jatin/Documents/drone_security_lab/attacks python3 attack4_replay.py

Detector output:

[ATTACK-4 Replay Attack] sig_ts 86340.0s behind wall clock Rule fired: SIG_PAST (offset=-86340.0s < threshold -60s) Confidence: RULE 100%

[ SCREENSHOT 7: Detector alerting Attack 4 ]

5.  Part 3 — Running the Defense

The defense proxy sits between the attacker and the drone. It validates every signed packet through 4 layers before forwarding it.

| Layer | Check | Catches |
| --- | --- | --- |
| L1 | ¦signing_ts − wall_clock¦ < 2 s | Attack 1 (future ts), Attack 4 (past ts) |
| L2 | signing_ts > last_accepted[link_id] | Attack 4 (stale replay after reset) |
| L3 | ¦signing_ts − gps_ts¦ < 5 s | Attack 1 if wall clock also compromised |
| L4 | Replay cache (SHA-256 fingerprint) | Attack 4 (repeated identical packets) |

Terminal 4 — Start defense proxy

Step 10  Start the proxy

cd /home/jatin/Documents/drone_security_lab/defense python3 secure_timestamp_defense.py \ --listen 14549 \ --forward 14550 \ --gps 25101 \ --verbose

Note: Keep this terminal visible. All PASS/BLOCK output appears here.

Proxy startup output:

[DEFENSE] Secure Timestamp Proxy Listening for packets : 0.0.0.0:14549 Forwarding valid to : 127.0.0.1:14550 GPS time feed : :25101 L1 tight window : ±2.0s L3 GPS cross-check : ±5.0s L4 replay cache size : 2000

[ SCREENSHOT 8: Defense proxy startup banner ]

Terminal 5 — Test normal traffic

Step 11  Send 5 normal signed packets

python3 /home/jatin/Documents/drone_security_lab/defense/test_normal.py

Expected in Terminal 4 (proxy):

[HH:MM:SS] PASS ok [HH:MM:SS] PASS ok [HH:MM:SS] PASS ok (x5 total)

[ SCREENSHOT 9: Normal traffic — all PASS ]

Test Attack 1 against the defense

Step 12  Send 10 future-timestamp packets

python3 /home/jatin/Documents/drone_security_lab/defense/test_attack1.py

Expected in Terminal 4 (proxy):

[HH:MM:SS] BLOCK L1-TIGHT-WINDOW sig_ts +86400.000s from wall clock (FUTURE, threshold ±2.0s) [HH:MM:SS] BLOCK L1-TIGHT-WINDOW sig_ts +86400.000s ... (x10 total)

[ SCREENSHOT 10: Attack 1 — all 10 BLOCKED by L1 ]

Test Attack 4 against the defense

Step 13  Send 10 replay packets (same packet repeated)

python3 /home/jatin/Documents/drone_security_lab/defense/test_attack4.py

Expected in Terminal 4 (proxy):

[HH:MM:SS] PASS ok [HH:MM:SS] BLOCK L4-REPLAY fingerprint abc123... already seen [HH:MM:SS] BLOCK L4-REPLAY fingerprint abc123... (x9 more)

[ SCREENSHOT 11: Attack 4 — copy 1 passes, copies 2-10 BLOCKED by L4 ]

View defense summary

Press Ctrl-C in Terminal 4. You will see:

================================================== DEFENSE SUMMARY Total packets seen : 25 Unsigned (forwarded) : 0 Signed passed : 6 Blocked (total) : 19 L1 Tight window : 10 L4 Replay cache : 9 Avg validation latency: 4.1 µs ==================================================

[ SCREENSHOT 12: Defense summary after Ctrl-C ]

Standalone Evaluation (no SITL needed)

Step 14  Run all 7 scenarios automatically

cd /home/jatin/Documents/drone_security_lab/defense python3 test_defense.py

Final metrics output:

False Acceptance Rate (FAR) : 0.00% (0 normal packets blocked) True Positive Rate (TPR) : 99.67% (299/300 attack packets blocked) Avg validation latency : 4.4 µs (< 1 ms — negligible overhead) MAVLink 2.0 compatibility : 100% (unsigned always forwarded)

[ SCREENSHOT 13: Full test_defense.py output ]

6.  Key Code Snippets

6.1  Rule-based detection (realtime_detector_v4.py)

Rules run first with 100% confidence. Priority order: Attack 3 → 2 → 1 → 4

def rule_classify(sig_ts, arrival, gps_mismatch): # Attack 3: GPS overflow / DoS if gps_mismatch is not None and gps_mismatch > ATTACK3_GPS_THRESHOLD: return 3, 100.0, 'GPS_OVERFLOW' # Attack 2: GPS spoofing if gps_mismatch is not None and gps_mismatch > GPS_MISMATCH_MIN: return 2, 100.0, 'GPS_MISMATCH' # Attack 1: future timestamp injection if sig_ts is not None and (sig_ts - arrival) > SIG_FUTURE_MIN: return 1, 100.0, 'SIG_FUTURE' # Attack 4: replay (past timestamp) if sig_ts is not None and (arrival - sig_ts) > SIG_PAST_MIN: return 4, 100.0, 'SIG_PAST' return None, 0.0, '' # no rule fired → pass to ML

6.2  ML fallback classification (realtime_detector_v4.py)

Random Forest with 10 engineered features. Confidence threshold = 70%.

features = extract_features(sig_ts, arrival, gps_mismatch, prev_sig, ...) probs = model.predict_proba([features])[0] pred = int(np.argmax(probs)) conf = probs[pred] * 100 if conf < ML_CONF_MIN: # 70% threshold pred, conf = 0, conf # treat as Normal

6.3  Defense proxy — 4-layer validation (secure_timestamp_defense.py)

Each signed packet passes through all 4 layers sequentially.

def validate(self, pkt): if not pkt['signed']: return True, 'unsigned' # always forward sig_ts = pkt['sig_ts'] link_id = pkt['link_id'] wall = time.time() # L1: tight window — must be within ±2s of wall clock if abs(sig_ts - wall) > self.tight_window: return False, 'L1-TIGHT-WINDOW ...' # L2: monotonic — must not go backwards last = self._last_accepted.get(link_id) if last and sig_ts < (last - 0.1): return False, 'L2-MONOTONIC ...' # L3: GPS cross-check — must match GPS time within 5s if self._gps_ts and abs(sig_ts - self._gps_ts) > self.gps_window: return False, 'L3-GPS-CROSS ...' # L4: replay cache — fingerprint must not be seen before fp = self._fingerprint(pkt) if fp in self._replay_cache: return False, 'L4-REPLAY ...' self._last_accepted[link_id] = sig_ts self._replay_cache[fp] = None return True, 'ok'

6.4  MAVLink v2 signing timestamp extraction

The signing block is the last 13 bytes of a signed MAVLink v2 packet.

# MAVLink v2 signed packet layout: # [magic][len][incompat=0x01][compat][seq][sysid][compid][msg_id x3] # [payload x len][CRC x2][link_id][ts x6][sig x6] ← signing block sig_block = data[10 + plen + 2 : 10 + plen + 2 + 13] link_id = sig_block[0] ts_raw = int.from_bytes(sig_block[1:7], 'little') # 10µs units sig_ts = MAVLINK_EPOCH + ts_raw / 1e5 # unix seconds # MAVLINK_EPOCH = 1,420,070,400 (Jan 1 2015 UTC)

7.  Results Summary

7.1  IDS Detection Results

| Attack | Method | Detection Time | Accuracy |
| --- | --- | --- | --- |
| Attack 1 — Timestamp Injection | Rule: SIG_FUTURE > 60s | < 50 ms | 100% |
| Attack 2 — GPS Spoofing | Rule: GPS_MISMATCH > 300s | < 50 ms | 100% |
| Attack 3 — Overflow / DoS | Rule: GPS > 1,000,000,000s | < 50 ms | 100% |
| Attack 4 — Replay Attack | Rule: SIG_PAST > 60s | < 50 ms | 100% |
| Normal traffic | ML: Random Forest fallback | < 50 ms | 100% |

7.2  ML Model Performance

| Metric | Value |
| --- | --- |
| Algorithm | Random Forest (200 trees) |
| Cross-validation folds | 5-fold stratified |
| F1 score (all folds) | 1.0000 |
| Top feature | F4: gps_system_mismatch_sec (26.44% importance) |
| 2nd feature | F8: signing_ts_abs_offset (21.44% importance) |
| ML confidence threshold | 70% — below this treated as Normal |

7.3  Defense Evaluation Results

| Metric | Result | Requirement |
| --- | --- | --- |
| False Acceptance Rate (FAR) | 0.00 % | < 1 % |
| True Positive Rate (TPR) | 99.67 % | > 99 % |
| Avg validation latency | 4.4 µs | < 1 ms |
| MAVLink 2.0 compatibility | 100 % | 100 % |
| Hardware modification needed | None | None |

8.  Stop Everything

Step 15  Kill SITL, Gazebo, MAVProxy

python3 /home/jatin/Documents/drone_security_lab/simulation/sitl_basic.py --kill

Also press Ctrl-C in all other terminals.

9.  File Reference

| File | Location | Purpose |
| --- | --- | --- |
| sitl_basic.py | simulation/ | Launches SITL + Gazebo + MAVProxy + Wireshark |
| simulate_normal_gps.py | drone_security_lab/ | Feeds normal GPS data to detector |
| realtime_detector_v4.py | mavlink-detection/ | Main IDS — rule + ML hybrid detector |
| train_random_forest.py | mavlink-detection/ | Trains the RF model on feature CSVs |
| attack1_signed_cmd.py | attacks/ | Attack 1: future timestamp injection |
| attack2_gps_spoof.py | attacks/ | Attack 2: GPS coordinate spoofing |
| attack3_overflow_dos.py | attacks/ | Attack 3: GPS timestamp overflow |
| attack4_replay.py | attacks/ | Attack 4: replay of captured packet |
| custom-input.py | home/jatin/ | Manual signed packet injector |
| secure_timestamp_defense.py | defense/ | Defense proxy — 4-layer validator |
| test_defense.py | defense/ | Standalone evaluation harness |
| test_normal.py | defense/ | Sends 5 normal packets to proxy |
| test_attack1.py | defense/ | Sends 10 future-ts packets to proxy |
| test_attack4.py | defense/ | Sends 10 replay packets to proxy |