Internship Daily Report — Summary

# May 27 — Environment Setup

Goal: Stand up the ArduPilot SITL simulation environment.

- Built ArduPilot SITL and ArduCopter firmware on Arch Linux (patched for GCC 16); set up a Python venv with OpenCV and MAVProxy.

- Installed MAVProxy and PyMAVLink; verified SITL launch with the Map and Console modules.

- Gazebo Classic failed to install on Arch (broken dependencies) — decided to move to Ubuntu 22.04 for stable support.

# May 28 — Full Simulation Stack + First Flight

Goal: Run SITL, Gazebo, MAVProxy, and Wireshark from a single launcher.

- Wrote sitl_basic.py to launch all four tools in order (SITL → Gazebo → MAVProxy → Wireshark), with --kill and --no-gz options.

- Fixed the SITL/Gazebo lock_step deadlock, enabled JSON FDM (--model JSON), and cleared a stale eeprom.bin frame bug.

- Generated a MAVLink Lua dissector so Wireshark decodes live HEARTBEAT, ATTITUDE, and position packets.

- First successful flight: GUIDED → arm → takeoff to 10 m, hovered, visible in the Gazebo 3D view.

# May 29 — First Two Spoofing Attacks

Goal: Reproduce the first two MAVLink timestamp attacks from the IEEE paper.

- Organised the drone_security_lab/ project folder with one script per attack.

- Attack 1: sent a signed LAND command with a +1-day future timestamp (765 packets captured).

- Attack 2: injected fake GPS_INPUT messages at +10 days (1,596 packets, 150 of them attack packets).

- Added a custom Wireshark field to display spoofed timestamps as readable dates.

# June 1 — All Four Attacks on MAVLink 2.0

Goal: Implement all four attacks with the correct MAVLink 2.0 wire format.

- Forced MAVLink 2.0 (magic byte 0xFD) in both scripts via the MAVLINK20 env var.

- Attack 3 (Overflow DoS): set GPS time to the 48-bit maximum — all real packets permanently rejected.

- Attack 4 (Replay): replayed a signed packet after a reboot with no key rotation — the UAV accepted it.

- Fixed corrupt pcaps by forcing the legacy -F pcap format and a clean shutdown.

# June 2 — Reference Alignment + Visible Effects

Goal: Align scripts with the reference implementation and make effects visible in Gazebo.

- Switched to manual SHA-256 signing and JSON-based GPS injection (matching the reference code).

- Attack 2 now drifts the reported position 200 m, so the drone visibly flies north in Gazebo.

- Added Attack 3B (Underflow DoS) using a pre-2015 timestamp.

- Built Wireshark Lua dissectors that decode every attack in the GUI, plus a full live Attack 4 replay demo in Gazebo.

# June 3 — Defensive Phase: Detection Framework

Goal: Design a real-time anomaly detection system for all five attacks.

- Defined six per-packet features: timestamp gap, GPS/system clock delta, position drift rate, inter-arrival time, sequence jump, and duplicate-hash flag.

- Layer A — six deterministic threshold rules that catch all five known attacks with no training data.

- Layer B — an unsupervised Isolation Forest that learns normal traffic and flags outliers (score > 0.6).

- Planned a three-thread detector and a five-step build order; an LSTM model is planned for Phase 2.

# June 4 — Feature Extraction Module

Goal: Implement the six-feature extraction engine for real-time packet analysis.

Created detection/features.py with the FeatureExtractor class covering ts_gap, iat_ms, seq_jump, gps_sys_delta, drift_m_per_s, and is_duplicate.

Implemented a SHA-256 rolling hash store (10,000 entries) for duplicate detection across the session.

Wrote binary parsers for MAVLink 2.0 headers and JSON parsers for GPS_INPUT strings; both return a unified Features dataclass.

Ran built-in self-tests: all six features computed correctly for normal, ATK2 spoof, ATK3 overflow, ATK3B underflow, and replay scenarios.

# June 5 — Statistical Rule Engine (Layer 1)

Goal: Build six deterministic threshold rules that catch every known attack instantly.

Implemented RuleEngine in detection/detector.py with rules for future timestamp (ATK1), GPS underflow (ATK3B), GPS overflow (ATK3), GPS clock spoof (ATK2), replay duplicate (ATK4), and burst DoS (ATK3B_BURST).

Configured thresholds: TS_GAP_FUTURE_SEC = 3,600 s; GPS_SPOOF_DELTA_SEC = 86,400 s; BURST_COUNT = 5 packets in a 2-second window.

All six rules fired correctly on crafted test packets; zero false positives on normal SITL traffic.

Integrated color-coded alert output in detection/alerts.py with per-session timestamped log files.

# June 8 — Detector Threading + Isolation Forest Baseline

Goal: Build the multi-threaded detector and train an unsupervised baseline model.

Added four UDP listener threads (ports 14550, 14551, 25100, 25101) feeding a shared queue; main loop dequeues and processes each packet.

Trained Isolation Forest on 5 minutes of normal SITL traffic; model saved to detection/models/baseline.pkl.

Discovered SO_REUSEPORT splits packets between sockets: heartbeats arrive on 14550 OR 14551, not both, causing false ATK4_REPLAY positives.

Fixed replay rule to fire only for monitor port 14551 MAVLink packets, eliminating false positives from SO_REUSEPORT duplication.

# June 9 — Detector Integration Testing

Goal: Test the full two-layer detector against all five attack scripts.

Ran detector.py alongside SITL and injected each attack: Layer 1 rules caught ATK1, ATK2, ATK3, ATK3B, and ATK4 correctly.

Isolation Forest correctly scored attack packets as anomalies (decision_function > 0.6) but could not name the attack class.

Identified the key limitation: Isolation Forest is binary (normal vs. anomaly) with no per-class labels.

Documented detection latency: all Layer 1 alerts fire within one packet arrival, under 1 ms per packet.

# June 10 — Debugging + Formula Derivation

Goal: Fix edge cases and document the mathematical basis for each detection feature.

Fixed GPS duplicate false positive: GPS_INPUT mirrors to both 25100 and 25101; added source == 'mavlink' filter to ATK4_REPLAY rule.

Derived exact feature formulas: ts_gap = sig_timestamp_unix − time.time(); gps_sys_delta = gps_unix − time.time().

Documented MAVLink epoch conversion: ts_val = (ts_unix − 1,420,070,400) × 100,000 (10 µs units since January 1 2015).

Created detection/detector_ml.py as the upgraded detector shell, ready to replace the Isolation Forest with a supervised classifier.

# June 11 — Detection Confirmed + Reports

Goal: Confirm end-to-end detection for all five attacks and produce written reports.

All five attacks produced correct named Layer 1 alerts; throttling in alerts.py suppresses repeated alerts within a 5-second window.

Verified the alert log written to detection/results/alerts_<timestamp>.log with a symlink alerts.log always pointing to the latest session.

Wrote formula_derivation_report.docx explaining ts_gap, gps_sys_delta, and 48-bit timestamp math in detail.

Pushed daily work report (daily_report_june11.docx) and all detection code to GitHub.

# June 12 — ML Upgrade Planning

Goal: Plan migration from Isolation Forest to a supervised RandomForest classifier.

Identified three limitations of Isolation Forest: binary output only, no named class labels, and iat_ms/seq_jump corrupted by SO_REUSEPORT.

Chose three clean classification features: ts_gap, gps_sys_delta, is_duplicate — all computable without SO_REUSEPORT interference.

Selected RandomForestClassifier (200 trees) as replacement: supports six named classes and predict_proba() for confidence scores.

Designed the synthetic dataset approach: Gaussian distributions anchored to exact attack-script constants, avoiding the need for live SITL traffic.

# June 15 — Synthetic Dataset Design

Goal: Design the six-class labeled training dataset for the RandomForest classifier.

Defined class means from attack constants: ATK1/ATK4 ts_gap = +86,400 s; ATK2 gps_sys_delta = +864,000 s; ATK3 gps_sys_delta ≈ +2.48 billion s; ATK3B gps_sys_delta ≈ −488 million s.

Set Gaussian noise: σ = 2.0 s for normal packets, σ = 50.0 s for attack packets to simulate real sensor jitter.

Planned 1,000 samples per class (6,000 total), 80/20 stratified train/test split.

Started ml/generate_dataset.py with class definitions and noise parameters.

# June 16 — Dataset Generation

Goal: Complete and run the synthetic dataset generator; verify class separation.

Finished ml/generate_dataset.py; generated ml/dataset/train.csv (4,800 rows) and ml/dataset/test.csv (1,200 rows).

Verified class separation: ATK3 and ATK3B means are billions of seconds from normal, guaranteeing linear separability in feature space.

Added ml/dataset/meta.txt recording reference_time_unix so ATK3/ATK3B means can be reproduced at any future time.

Confirmed the dataset is balanced: exactly 800 train samples and 200 test samples per class.

# June 17 — Classifier Training

Goal: Train, evaluate, and save the RandomForest attack classifier.

Implemented ml/train_classifier.py with a StandardScaler + RandomForestClassifier(200 trees) sklearn Pipeline.

Trained on train.csv and evaluated on test.csv: 100% accuracy across all six classes.

Feature importances: gps_sys_delta = 53%, ts_gap = 27%, is_duplicate = 20%.

Saved the model bundle to detection/models/classifier.pkl as {model, features, label_names} for use by the detector.

# June 18 — RandomForest Integration into Detector

Goal: Replace the Isolation Forest layer with the trained RandomForest classifier.

Replaced MLLayer with ClassifierLayer in detection/detector_ml.py; changed MODEL_FILE to classifier.pkl.

Added ML_CONF_THRESHOLD = 0.70; ML alerts fire only when Layer 1 rules did not already catch the packet.

Alert format: ATK_ML_<CLASS> tag with confidence percentage and raw feature vector logged.

Updated _load_model() to unpack the classifier.pkl bundle (model, features, label_names).

# June 19 — Standalone Attack Injector

Goal: Build a test tool that injects crafted attacks without requiring SITL.

Created tests/inject_test.py sending crafted packets directly to monitor ports 14551 (MAVLink) and 25101 (GPS).

Built _build_mavlink_signed() constructing minimal valid MAVLink 2.0 signed packets; the detector does not validate CRC or signature hash.

Built _build_gps_json() producing GPS_INPUT JSON matching the MAVProxy GPSInput format.

Injector supports: normal, atk1, atk2, atk3, atk3b, atk4, and all arguments.

# June 22 — Injector Bug Fix + End-to-End Test

Goal: Fix the injector argument bug and confirm all attacks are detected correctly.

Fixed bug in inject_test.py: script only read sys.argv[1] (one attack); updated to iterate sys.argv[1:] to support multiple arguments.

Ran inject_test.py all with the detector running: all five attacks and the burst pattern detected correctly by Layer 1 rules.

Confirmed Layer 2 ML fires only for packets that bypass the statistical rules; no double alerts.

Conducted prerequisite knowledge review covering MAVLink structure, sklearn Pipeline, and Python UDP threading.

# June 23 — Code Quality Fixes

Goal: Fix stale code left over from the Isolation Forest era and push clean commits.

Fixed ml_vector in detection/features.py: was returning 2 features (Isolation Forest remnant); updated to 3 features [ts_gap, gps_sys_delta, is_duplicate].

Updated ClassifierLayer.classify() in detector_ml.py to call feat.ml_vector instead of duplicating the array inline.

Added root-level mav.parm, mav.tlog, mav.tlog.raw to .gitignore; these files appear when ArduPilot runs from the repository root.

Pushed fixes to GitHub in four separate commits, one logical change per commit.

# June 24 — Defense Module

Goal: Implement a countermeasure against ATK1 (future timestamp) and ATK4 (replay).

Created defense/secure_timestamp_defense.py with a SecureTimestampDefense class that validates incoming MAVLink packets.

Defense rejects any packet with a signing timestamp more than 1 second in the future or matching a recent-hash replay cache.

Replay cache maintains a rolling 1,000-entry SHA-256 hash store; first occurrence is accepted, subsequent duplicates are blocked.

Created four defense test scripts: test_normal.py, test_attack1.py, test_attack4.py, and test_defense.py.

# June 25 — Defense Testing

Goal: Validate the defense module against all attack scenarios and confirm no false positives.

test_attack1.py: defense correctly rejected all 50 packets with future timestamps (100% block rate).

test_attack4.py: defense blocked 49 of 50 replayed packets; the first occurrence is accepted by design (98% block rate).

test_normal.py: defense passed all 50 legitimate packets with zero false positives (100% pass rate).

test_defense.py confirmed all three scenarios in a single run; results consistent across five repeated runs.

# June 26 — Defense Evaluation + Results

Goal: Measure quantitative defense performance and write the evaluation report.

Generated defense/results/defense_eval.csv with columns: scenario, total_packets, accepted, blocked, block_rate.

Normal scenario: 50 packets, 50 accepted, 0 blocked (100% pass rate).

ATK1 scenario: 50 packets, 0 accepted, 50 blocked (100% block rate).

ATK4 scenario: 50 packets, 1 accepted (first packet), 49 blocked (98% block rate); wrote defense_report.docx summarizing methodology and results.

# June 29 — Final Reports

Goal: Generate comprehensive project reports covering all phases of the internship.

Produced reports/mavlink_ids_full_report.docx covering all three phases: attack replication, real-time detection, and defense.

Produced reports/mavlink_ids_FINAL.docx as the polished submission version.

Generated run_guide.docx with step-by-step instructions for running SITL, the detector, and the injector.

Pushed all new reports to GitHub in separate commits.

# June 30 — Custom Attack Script + Final Captures

Goal: Add reference attack script and capture final pcap results for all attacks.

Added attacks/custom-input.py: low-level MAVLink signing attack matching the reference paper implementation exactly.

Re-ran all four attack scripts against the live SITL and saved updated pcap captures to attacks/results/.

Verified all pcap files open correctly in Wireshark and display the correct MAVLink protocol decode.

Pushed the custom-input.py script and all updated captures to GitHub.

# July 1 — Project Cleanup

Goal: Final code review, repository cleanup, and full pipeline verification.

Reviewed all Python files for syntax errors using ast.parse(); all files pass cleanly.

Confirmed .gitignore covers all runtime-generated files: models (*.pkl), logs (*.log), and ArduPilot runtime files.

Tested the full pipeline from scratch: generate_dataset.py → train_classifier.py → detector_ml.py → inject_test.py all — all alerts fired correctly.

Repository is clean: git status shows no untracked files outside expected generated artifacts.

# July 2 — Defense Report and Run Guide Update

Goal: Finalize defense evaluation report and update run guide documentation.

Updated defense/generate_defense_report.py to produce a formatted Word document with evaluation tables and block-rate percentages.

Updated run_guide.docx to include defense testing instructions alongside existing detector and injector steps.

Pushed generate_defense_report.py and the updated run_guide.docx to GitHub.

Verified the defense module handles the boundary case correctly: packets timestamped exactly at the 1-second threshold are accepted.

# July 3 — Final Documentation Review

Goal: Ensure all documentation is complete, consistent, and matches the code.

Reviewed all reports for consistency: attack names, feature formulas, and threshold values match the codebase exactly.

Confirmed detection/results/alerts.log captures all six alert types (ATK1_FUTURE_TS, ATK2_GPS_SPOOF, ATK3_OVERFLOW, ATK3B_UNDERFLOW, ATK3B_BURST, ATK4_REPLAY) with correct timestamps.

Updated daily_report_june22.docx and daily_report_june24.docx with finalized content.

All GitHub commits verified: 20+ commits from initial environment setup through defense evaluation.

# July 6 — Project Review

Goal: End-to-end review of the complete project and repository before final submission.

Walked through the entire codebase confirming no dead code or stale references remain.

Confirmed the two-layer detection architecture: Layer 1 (six statistical rules) catches all known attacks instantly; Layer 2 (RandomForest) provides named classification for edge cases.

Updated simulation/mav.parm from the latest SITL session and re-captured pcap files for attacks 1–4.

Pushed final pcap updates and simulation parameters to GitHub.

# July 7 — Final GitHub Push

Goal: Push all remaining updates and confirm the repository is complete.

Staged and pushed final attack captures (attacks 1–4 updated pcap files) and mav.parm to GitHub.

Repository final state: 5 attack scripts, 1 defense module with 4 test scripts, 1 two-layer detector, 1 standalone injector, and 8 report documents.

Confirmed the GitHub repository at JatinThakur2/Mavlink-Drone-Attack is fully up to date.

Project handover documentation complete and verified.

# July 8 — Internship Wrap-Up

Goal: Complete the final internship report, verify all deliverables, and submit.

Reviewed all daily reports (May 27 – July 8) for completeness and accuracy; added missing daily entries to the summary document.

Final system verification: all five attacks detected by the two-layer detector; defense blocks ATK1 and ATK4 at 100% and 98% respectively.

Documented key learnings: MAVLink 2.0 signing protocol, Python UDP socket threading, supervised vs. unsupervised ML, and feature engineering for protocol security.

Internship deliverables submitted: source code on GitHub, attack pcap captures, detection reports, and defense evaluation results.

# Attack Summary

| Attack | Method | Effect |
| --- | --- | --- |
| 1 — Clock Manipulation | Signed command with +1-day timestamp | UAV clock shifted forward, LAND triggered |
| 2 — GPS Spoof | GPS_INPUT at +10 days + position drift | Clock +10 days; drone flies 200 m north |
| 3 — Overflow DoS | GPS time at the 48-bit maximum (year 2104) | All signed packets permanently rejected |
| 3B — Underflow DoS | GPS time before the 2015 epoch (year 2010) | DoS or clock reset (implementation dependent) |
| 4 — Replay | Resend signed bytes after reboot, no key rotation | Replayed command accepted (COMMAND_ACK) |