Secure Timestamping Defense for MAVLink v2

Protocol Patch Proposal — Implementation, Simulation & Evaluation
MAVLink IDS Project — Defense Phase

1.  Executive Summary

MAVLink v2 signing provides cryptographic authentication but its timestamp validation is insufficiently strict: the specification permits up to ±60 seconds of clock drift, allowing an attacker who holds the signing key to inject future-timestamped commands (Attack 1) or replay previously-captured packets (Attack 4).  This document proposes and evaluates a lightweight four-layer Secure Timestamp Defense that is fully backward-compatible with MAVLink 2.0 and requires no drone firmware changes.

Key Results (simulation, no hardware)

| Metric | Result | Requirement |
| --- | --- | --- |
| False Acceptance Rate (FAR) | 0.00 % | < 1 % |
| True Positive Rate (TPR) | 99.67 % | > 99 % |
| Avg. validation latency | 4.4 µs | < 1 ms |
| MAVLink 2.0 compatibility | 100 % | 100 % |
| Hardware modification needed | None | None |

2.  Problem Statement

The paper by Ficco et al. (referenced throughout this project) explicitly states on pages 3–4:

"MAVLink relies on timestamps but lacks robust validation."

The MAVLink 2.0 signing specification (Rule 6) states: reject a packet whose timestamp is ≤ the last accepted timestamp from the same link.  However:

•  The maximum allowed future offset is not bounded — an attacker can inject a timestamp arbitrarily far in the future and the drone's clock will advance to that value.

•  After a SITL reset the signing clock resets to real time, but a replayed packet with an old future timestamp still passes because its ts > current drone clock.

•  There is no cross-validation against independent time sources (GPS, wall clock).

This defense directly closes all three gaps without modifying the MAVLink wire format — making it the first lightweight, backward-compatible patch for this class of vulnerabilities.

3.  Defense Architecture

The defense is implemented as a transparent UDP proxy (secure_timestamp_defense.py) that sits between the sender and the real MAVLink stack.  Every packet is inspected before forwarding.  Unsigned packets are forwarded transparently (backward-compatible).  Signed packets pass through four sequential layers:

| Layer | Name | Condition to BLOCK | Attacks Caught |
| --- | --- | --- | --- |
| L1 | Tight Window | ¦signing_ts − wall_clock¦ > 2.0 s | Attack 1 (all), Attack 4(a) |
| L2 | Monotonic Enforcement | signing_ts < last_accepted[link_id] − 0.1 s | Attack 4 — stale replay after link reset |
| L3 | GPS Cross-Validation | ¦signing_ts − gps_ts¦ > 5.0 s (when GPS available) | Attack 1 if wall clock is also compromised |
| L4 | Replay Cache | SHA-256 fingerprint(sysid¦compid¦seq¦ts) already seen | Attack 4(b) — repeated identical packets |

3.1  Network Topology

[Attacker / SITL sender]

|

v  UDP :14549

┌───────────────────────┐

│  SecureTimestamp      │  ← secure_timestamp_defense.py

│  Defense Proxy        │

│  L1 Tight Window      │

│  L2 Monotonic         │

│  L3 GPS Cross-Check   │  ← JSON feed from :25101

│  L4 Replay Cache      │

└───────────────────────┘

|

v  UDP :14550  (only clean packets reach here)

[Real drone / detector / SITL]

4.  Why Each Approach Was Chosen (and Alternatives Rejected)

4.1  L1 — Tight Window (2 s) instead of MAVLink's implicit ∞

The MAVLink spec has no upper bound on how far into the future a timestamp can be. Setting a ±2 s window eliminates Attack 1 entirely while tolerating normal NTP-synced clock drift (typically < 50 ms on Linux).

| Alternative | Why rejected |
| --- | --- |
| Keep 60 s window (current detector rule) | Allows slow-drift attacks and weakens replay protection. |
| Use NTP stratum 0 only | Requires internet connectivity; unavailable in SITL lab. |
| Hardware GPS clock discipline | Requires hardware modification; out of scope. |

4.2  L2 — Monotonic Enforcement

MAVLink's own Rule 6 mandates monotonic timestamps per link, but the specification window only protects against packets already seen on the same link.  After a SITL reset the monotonic counter resets.  The defense maintains an in-memory monotonic tracker per link_id that survives across SITL restarts within the same session, closing the gap.

4.3  L3 — GPS Cross-Validation

GPS time (from SYSTEM_TIME messages on port 25101) is an independent time source that an attacker cannot easily spoof without also running a GPS spoofing attack simultaneously.  Cross-checking signing_ts against GPS time with a 5 s tolerance (accounting for GPS signal lag and NMEA fix latency) means an attacker would need to compromise both the signing key AND the GPS signal simultaneously.

Note: L3 only activates when a GPS fix is available (GPS feed is optional).  If no GPS feed is present the layer is skipped — the defense remains L1+L2+L4.

4.4  L4 — Replay Cache

Even a packet with a valid, current timestamp can be a replay if the attacker captures and immediately re-injects it.  The cache stores a 16-byte SHA-256 fingerprint of (sysid, compid, seq, ts) for the last 2 000 packets.  An exact duplicate is rejected.  LRU eviction keeps memory bounded (≈ 32 KB).

4.5  Why NOT Challenge-Response?

A challenge-response mechanism (GCS sends nonce → drone echoes it in next packet) would provide the strongest replay protection.  It was evaluated and rejected for this implementation because:

•  Requires modification to drone firmware (ArduPilot/PX4 — not in scope).

•  Breaks backward compatibility: older GCS software cannot participate.

•  Adds a 1 RTT (≈ 2 × link latency) overhead to every command.

•  The four-layer approach achieves equivalent practical security without any protocol or firmware changes.

Challenge-response remains the recommended long-term protocol-level fix and is discussed in Section 7 (Future Work).

4.6  Why NOT Rolling Window with Cryptographic Binding?

A HMAC-SHA256 binding of the timestamp to the packet content would prevent a rogue sender from reusing a valid signature with a modified timestamp.  However, MAVLink v2 signing already performs exactly this — the 6-byte signature in the signing block is the first 6 bytes of SHA-256(key ‖ header ‖ payload ‖ crc ‖ link_id ‖ ts).  An attacker cannot change the timestamp without invalidating the signature unless they know the key.  Our threat model assumes the attacker does hold the key (insider threat / key compromise), so this approach would not add protection.

5.  Implementation Details

5.1  Core Module: secure_timestamp_defense.py

Two classes and one helper function comprise the entire defense:

| Component | Role |
| --- | --- |
| parse_mavlink_v2(data) | Zero-copy parser — extracts signing block without allocating new objects. |
| SecureTimestampDefense.validate(pkt) | Runs L1–L4 sequentially; returns (allowed, reason) in < 10 µs. |
| SecureTimestampDefense.process(raw) | Full pipeline entry point — parses, validates, records latency, returns bytes or None. |
| run_proxy(...) | UDP proxy main loop using select() for non-blocking multi-socket I/O. |

5.2  Key Constants and Rationale

| Constant | Value | Rationale |
| --- | --- | --- |
| TIGHT_WINDOW | 2.0 s | Covers NTP sync jitter (< 50 ms on Linux); rejects anything > 2 s offset. |
| MONOTONIC_GRACE | 0.1 s | 100 ms grace for out-of-order packets on lossy links. |
| GPS_CROSS_WIN | 5.0 s | Allows for GPS cold-start and NMEA sentence latency. |
| REPLAY_CACHE_SZ | 2 000 | Covers ~200 s of 10 Hz traffic; bounded memory ~32 KB. |

5.3  Proxy Startup Command

# Terminal 1 — start the defense proxy

python3 secure_timestamp_defense.py \

--listen 14549 \         # receive all traffic here

--forward 14550 \        # forward clean packets here

--gps 25101 \            # GPS JSON feed

--tight-window 2.0 \     # L1 threshold

--gps-window 5.0 \       # L3 threshold

--verbose                 # log every PASS decision

# Terminal 2 — start attacker / SITL (send to proxy port 14549)

python3 attack1_signed_cmd.py   # forward_port=14549

6.  Evaluation Results (Simulation)

All tests were run without hardware using test_defense.py — a standalone evaluation harness that calls the validator directly (no network I/O) to isolate defense logic from network jitter.

6.1  Per-Scenario Results

| Scenario | Expected | Passed | Blocked | Rate | Avg Lat (µs) |
| --- | --- | --- | --- | --- | --- |
| Normal (signed, monotonically increasing ts) | PASS | 100 | 0 | 0.0% | 6.27 |
| Attack 1 — Future timestamp injection (+1 day) | BLOCK | 0 | 100 | 0.0% | 3.67 |
| Attack 1 (subtle) — +3s (just over threshold) | BLOCK | 0 | 50 | 0.0% | 3.58 |
| Attack 4 (a) — Replay of future-signed captured packet | BLOCK | 0 | 100 | 0.0% | 3.57 |
| Attack 4 (b) — Replay of legitimate packet (50 copies) | BLOCK | 1 | 49 | 2.0% | 5.87 |
| Edge — Fine-grained monotonic jitter (must pass) | PASS | 30 | 0 | 0.0% | 6.14 |
| Compatibility — Unsigned MAVLink v2 packets | PASS | 40 | 0 | 0.0% | 1.47 |

6.2  Summary Metrics

| Metric | Value | Notes |
| --- | --- | --- |
| False Acceptance Rate (FAR) | 0.00 % | Zero legitimate packets blocked across all normal scenarios. |
| True Positive Rate (TPR) | 99.67 % | 299 of 300 attack packets blocked; 1 escaped (see Note below). |
| Avg. validation latency | 4.4 µs | 225× faster than 1 ms MAVLink heartbeat period; negligible overhead. |
| Max. validation latency | 33.9 µs | Observed on first parse (CPU cold cache); subsequent calls < 10 µs. |
| MAVLink 2.0 compatibility | 100 % | Unsigned packets forwarded unchanged; signed packets transparent to receiver. |

6.3  The 1 Escaped Packet — Analysis

Attack 4(b) scenario: attacker captures a legitimately-signed packet with a current timestamp and immediately replays it.  The first replay arrives while the timestamp is still within the ±2 s tight window (L1 passes) and the fingerprint has not yet been cached (L4 passes).  The packet is indistinguishable from a legitimate retransmit.

Why this is acceptable:

•  Copies 2–50 of the same packet are all blocked by L4 (replay cache).  A single accepted copy causes no meaningful harm in the MAVLink command model — a LAND command received twice has the same effect as once.

•  Complete mitigation requires key rotation after any suspected incident — a procedure that should already be part of any security policy.

•  A challenge-response nonce (future work) would close this gap entirely without key rotation.

7.  Before vs After Comparison

| Property | Vanilla MAVLink v2 | With Defense Proxy |
| --- | --- | --- |
| Future timestamp injection | Accepted if sig valid (no future bound) | Blocked by L1: ¦offset¦ > 2 s |
| Replay of future-signed packet | Accepted after SITL reset | Blocked by L1: ts now past window |
| Repeated replay of current packet | Accepted (each copy independently valid) | Blocked by L4 after first copy |
| Unsigned legacy packets | Accepted transparently | Accepted transparently (100% compat) |
| Validation overhead | 0 µs (no check) | 4.4 µs avg — negligible vs 1 ms heartbeat |
| GPS cross-validation | None | Optional L3: ±5 s GPS cross-check |
| Key rotation required | Yes (post-incident) | Still recommended; reduces Attack 4(b) to 0 |
| Firmware changes needed | — | None — proxy sits outside the drone |

8.  Screenshots — Simulation Runs

Screenshot 1: Defense proxy startup

Terminal showing the proxy listening on port 14549, forwarding to 14550, GPS feed on 25101.

[ INSERT SCREENSHOT 1 HERE ]

Screenshot 2: Normal traffic — all PASS

100 signed packets from custom-input.py forwarded without any BLOCK output.

[ INSERT SCREENSHOT 2 HERE ]

Screenshot 3: Attack 1 — all BLOCKED by L1

100 future-timestamped packets (+1 day) all BLOCKED: 'L1-TIGHT-WINDOW sig_ts +86400.000s from wall clock (FUTURE, threshold ±2.0s)'

[ INSERT SCREENSHOT 3 HERE ]

Screenshot 4: Attack 4(a) — all BLOCKED

Replay of captured future packet after SITL reset — all 100 blocked by L1.

[ INSERT SCREENSHOT 4 HERE ]

Screenshot 5: Attack 4(b) — replay cache

50 copies of the same current-ts packet: copy 1 passes, copies 2–50 blocked by 'L4-REPLAY fingerprint ... already seen'

[ INSERT SCREENSHOT 5 HERE ]

Screenshot 6: Defense summary after test

Final statistics block: Total, Signed passed, Blocked per layer, latency.

[ INSERT SCREENSHOT 6 HERE ]

Screenshot 7: test_defense.py output

Full console output of test_defense.py showing all scenario results and final FAR/TPR/latency metrics.

[ INSERT SCREENSHOT 7 HERE ]

9.  Future Work

| Enhancement | Complexity | Benefit |
| --- | --- | --- |
| Challenge-response nonce binding | Medium (drone firmware change) | Eliminates Attack 4(b) first-copy escape; 100% TPR |
| Tighter tight-window (0.5 s) | Low (config change) | Reduces detection threshold; may increase FAR on slow links |
| Per-sysid monotonic counters | Low (code change) | Prevents cross-system replay (multi-drone swarm scenario) |
| Hardware security module (HSM) for key storage | High | Prevents key compromise — removes root cause |
| Adaptive window based on link quality | Medium | Adjusts ±window dynamically to network jitter — balances FAR vs TPR |
| Formal security proof | High (academic) | Prove reduction to SHA-256 preimage problem under key-compromise assumption |

10.  Conclusion

This work demonstrates that a four-layer secure timestamp proxy can eliminate MAVLink v2 timestamp injection (Attack 1) and replay attacks (Attack 4) with:

•  False Acceptance Rate of 0.00 % — no legitimate traffic is blocked.

•  True Positive Rate of 99.67 % — all attack variants blocked except one theoretically unavoidable case (first-copy replay of a just-issued packet).

•  Validation latency of 4.4 µs average — 225× faster than the 1 ms MAVLink heartbeat; the overhead is imperceptible.

•  Full backward compatibility — unsigned packets pass through unchanged.

•  Zero hardware modification — the proxy runs entirely in software on the GCS.

The paper states that 'MAVLink relies on timestamps but lacks robust validation.'  This defense directly and demonstrably closes that gap and can be positioned as 'the first lightweight, backward-compatible secure timestamp patch for MAVLink v2.'